; Diagnostic control map

;

;  DIP switch:		Functions:

;  1 2 3 4 5 6 7 8	(0=on, 1=off, x=don't care)

;  ---------------

;  1 x 1 1 1 1 1 1	Normal operation (rah/wbh/no_rbchk)

;  1 x 1 0 1 1 x x	enable read after write check (rbchk, disables wbh)

;  1 x 1 x 1 1 0 x	disable write behind (wbh)

;  1 x 1 x 1 1 x 0	disable read ahead (rah)

;  0 0 x x x x x x	Tests done in single density

;  0 1 x x x x x x	Tests done in double density

;  0 x 0 0 x x x x	Format diskette, don't do readback check

;  0 x 0 1 x x x x	Format diskette, do readback check

;  0 x 1 x 0 x x x	Read all tracks/sectors sequentially

;  0 x 1 x 1 0 x x	Do random writes followed by random reads

;  0 x 1 x 1 x 0 x	Suspend random reads/writes

;  0 x 1 x 1 x 1 0	Read and write all 8 hardware I/O ports continiously

;  0 x 1 x 1 1 1 1	Read random sectors at random tracks

; Operation control map

;

; DIP switch:		Functions:

; 1 2 3 4		(0=on, 1=off)

; -------

; 1 x x x		Atari 815 mode (sectors 1-3 are 128 bytes long)

; 0 x x x		All sectors are 256 bytes long

; x 0 x 0		Seek speed 30 Ms/step (slow)

; x 0 x 1		Seek speed 20 Ms/step (not so slow)

; x 1 x 0		Seek speed 12 Ms/step (sort of fast)

; x 1 x 1		Seek speed  6 Ms/step (Pretty fast)

; x x 0 x		Multi-drive, odd numbered drives become 2 small drives

; x x 1 x		No multi-drive

; Note: Shugart 85x drives cannot run at the fastest seek speed.

; The default operational DIP switch setting should be:

; 1 2 3 4

; -------

; 1 0 0 1		815 mode/multi-drive selected

; The board is setup for radial select. Configure the drive selects as:

; Atari drive #		Drive selected

;	1			1

;	2			2

;	3			3

;	4			0