TITLE	'6545 KEYBOARD, EDITOR, AND SCREEN HANDLER'

SUBTTL	' '

LIST	-F,-M

SPACE	4,10

***	Copyright 1984 ATARI.  Unauthorized reproduction, adaptation,

*	distribution, performance or display of this computer program

*	or the associated audiovisual work is strictly prohibited.

SPACE	4,10

***	6545 Keyboard, Editor and Screen Handler

*

*	NOTES

*       MODS

*	The sections of the 6545 Keyboard, Editor and Screen Handler

*	appear in the following order:

*

*	Equates and Definitions

*

*		System Symbol Equates

*		System Address Equates

*		Miscellaneous Address Equates

*		Macro Definitions

*

*	Code and Data

*

LIST	-F,-M,-L,-R

SUBTTL	'System Symbol Equates'

SPACE	4,10

**	Assembly Option Equates

FALSE	EQU	0

TRUE	EQU	not FALSE

VGC	SET	TRUE	;virtual game controllers

RAMSYS	SET	FALSE	;not RAM based system

LNBUG	SET	FALSE	;no LNBUG interface

ACMI	SET	FALSE	;no asynchronous communications module interface

SPACE	4,10

**	Identification Equates

IDREV	EQU	$02	;identification revision number

IDDAY	EQU	$10	;identification day

IDMON	EQU	$05	;identification month

IDYEAR	EQU	$83	;identification year

IDCPU	EQU	$02	;identification CPU series

IDPN1	EQU	'B'	;identification part number field 1

IDPN2	EQU	'B'	;identification part number field 2

IDPN3	EQU	$00	;identification part number field 3

IDPN4	EQU	$00	;identification part number field 4

IDPN5	EQU	$01	;identification part number field 5

SPACE	4,10

**	Configuration Equates

*

*	NOTES

*		Problem: last byte of HATABS (as defined by MAXDEV)

*		overlaps first power-up validation byte.

MAXDEV	EQU	33	;offset to last possible entry of HATABS

IOCBSZ	EQU	16	;length of IOCB

SEIOCB	EQU	0*IOCBSZ	;screen editor IOCB index

MAXIOC	EQU	8*IOCBSZ	;first invalid IOCB index

DSCTSZ	EQU	128	;disk sector size

LEDGE	EQU	2	;left edge

REDGE	EQU	39	;right edge

INIML	EQU	$0700	;initial MEMLO

ICSORG	EQU	$CC00	;international character set origin

DCSORG	EQU	$E000	;domestic character set origin

SPACE	4,10

**	IOCB Command Code Equates

OPEN	EQU	$03	;open

GETREC	EQU	$05	;get record

GETCHR	EQU	$07	;get character(s)

PUTREC	EQU	$09	;put record

PUTCHR	EQU	$0B	;put character(s)

CLOSE	EQU	$0C	;close

STATIS	EQU	$0D	;status

SPECIL	EQU	$0E	;special

SPACE	4,10

**	Special Entry Command Equates

;	Screen Commands

DRAWLN	EQU	$11	;draw line

FILLIN	EQU	$12	;draw line with right fill

SPACE	4,10

**	ICAX1 Auxiliary Byte 1 Equates

APPEND	EQU	$01	;open write append (D:) or screen read (E:)

DIRECT	EQU	$02	;open for directory access (D:)

OPNIN	EQU	$04	;open for input (all devices)

OPNOT	EQU	$08	;open for output (all devices)

MXDMOD	EQU	$10	;open for mixed mode (E:, S:)

INSCLR	EQU	$20	;open for input without clearing screen (E:, S:)

SPACE	4,10

**	Device Code Equates

CASSET	EQU	'C'	;cassette

DISK	EQU	'D'	;disk

SCREDT	EQU	'E'	;screen editor

KBD	EQU	'K'	;keyboard

PRINTR	EQU	'P'	;printer

DISPLY	EQU	'S'	;screen display

SPACE	4,10

**	Character and Key Code Equates

CLS	EQU	$7D	;clear screen

EOL	EQU	$9B	;end of line (RETURN)

HELP	EQU	$11	;key code for HELP

CNTLF1	EQU	$83	;key code for CTRL-F1

CNTLF2	EQU	$84	;key code for CTRL-F2

CNTLF3	EQU	$93	;key code for CTRL-F3

CNTLF4	EQU	$94	;key code for CTRL-F4

CNTL1	EQU	$9F	;key code for CTRL-1

SPACE	4,10

**	Status Code Equates

SUCCES	EQU	1	;successful operation

BRKABT	EQU	128	;BREAK key abort

PRVOPN	EQU	129	;IOCB already open error

NONDEV	EQU	130	;nonexistent device error

WRONLY	EQU	131	;IOCB opened for write only error

NVALID	EQU	132	;invalid command error

NOTOPN	EQU	133	;device/file not open error

BADIOC	EQU	134	;invalid IOCB index error

RDONLY	EQU	135	;IOCB opened for read only error

EOFERR	EQU	136	;end of file error

TRNRCD	EQU	137	;truncated record error

TIMOUT	EQU	138	;peripheral device timeout error

DNACK	EQU	139	;device does not acknowledge command error

FRMERR	EQU	140	;serial bus framing error

CRSROR	EQU	141	;cursor overrange error

OVRRUN	EQU	142	;serial bus data overrun error

CHKERR	EQU	143	;serial bus checksum error

DERROR	EQU	144	;device done (operation incomplete) error

BADMOD	EQU	145	;bad screen mode number error

FNCNOT	EQU	146	;function not implemented in handler error

SCRMEM	EQU	147	;insufficient memory for screen mode error

**	DCB Device Bus ID Equates

DISKID	EQU	$31	;disk bus ID

PDEVN	EQU	$40	;printer bus ID

CASET	EQU	$60	;cassette bus ID

SPACE	4,10

**	Bus Command Equates

FOMAT	EQU	'!'	;format command

PUTSEC	EQU	'P'	;put sector command

READ	EQU	'R'	;read command

STATC	EQU	'S'	;status command

WRITE	EQU	'W'	;write command

SPACE	4,10

**	Command Auxiliary Byte Equates

DOUBLE	EQU	'D'	;print 20 characters double width

NORMAL	EQU	'N'	;print 40 characters normally

PLOT	EQU	'P'	;plot

SIDWAY	EQU	'S'	;print 16 characters sideways

SPACE	4,10

**	Bus Response Equates

ACK	EQU	'A'	;device acknowledged

COMPLT	EQU	'C'	;device successfully completed operation

ERROR	EQU	'E'	;device incurred error in attempted operation

NACK	EQU	'N'	;device did not understand

SPACE	4,10

**	Floating Point Package Miscellaneous Equates

FPREC	EQU	6	;precision

FMPREC	EQU	FPREC-1	;length of mantissa

SPACE	4,10

**	Cassette Record Type Equates

HDR	EQU	$FB	;header

DTA	EQU	$FC	;data record

DT1	EQU	$FA	;last data record

EOT	EQU	$FE	;end of tape (file)

TONE1	EQU	2	;record

TONE2	EQU	1	;playback

SPACE	4,10

**	Cassette Timing Equates

WLEADN	EQU	1152	;NTSC 19.2 second WRITE file leader

RLEADN	EQU	576	;NTSC 9.6 second READ file leader

WIRGLN	EQU	180	;NTSC 3.0 second WRITE IRG

RIRGLN	EQU	120	;NTSC 2.0 second READ IRG

WSIRGN	EQU	15	;NTSC 0.25 second WRITE short IRG

RSIRGN	EQU	10	;NTSC 0.16 second READ short IRG

BEEPNN	EQU	30	;NTSC 0.5 second beep duration

BEEPFN	EQU	10	;NTSC 0.16 second beep separation

WLEADP	EQU	960	;PAL 19.2 second WRITE file leader

RLEADP	EQU	480	;PAL 9.6 second READ file leader

WIRGLP	EQU	150	;PAL 3.0 second WRITE IRG

RIRGLP	EQU	100	;PAL 2.0 second READ IRG

WSIRGP	EQU	13	;PAL 0.25 second WRITE short IRG

RSIRGP	EQU	8	;PAL 0.16 second READ short IRG

BEEPNP	EQU	25	;PAL 0.5 second beep duration

BEEPFP	EQU	8	;PAL 0.16 second beep separation

WIRGHI	EQU	0	;high WRITE IRG

RIRGHI	EQU	0	;high READ IRG

SPACE	4,10

**	Power-up Validation Byte Value Equates

PUPVL1	EQU	$5C	;power-up validation value 1

PUPVL2	EQU	$93	;power-up validation value 2

PUPVL3	EQU	$25	;power-up validation value 3

SPACE	4,10

**	Relocating Loader Miscellaneous Equates

DATAER	EQU	156	;end of record appears before END record error

MEMERR	EQU	157	;memory insufficient for load error

SPACE	4,10

**	Miscellaneous Equates

IOCFRE	EQU	$FF	;IOCB free indicator

B19200	EQU	$0028	;19200 baud POKEY counter value

B00600	EQU	$05CC	;600 baud POKEY counter value

HITONE	EQU	$05	;FSK high freq. POKEY counter value (5326 Hz)

LOTONE	EQU	$07	;FSK low freq. POKEY counter value (3995 Hz)

NCOMLO	EQU	$34	;PIA lower NOT COMMAND line command

NCOMHI	EQU	$3C	;PIA raise NOT COMMAND line command

MOTRGO	EQU	$34	;PIA cassette motor ON command

MOTRST	EQU	$3C	;PIA cassette motor OFF command

NODAT	EQU	$00	;SIO immediate operation

GETDAT	EQU	$40	;SIO read data frame

PUTDAT	EQU	$80	;SIO write data frame

CRETRI	EQU	13	;number of command frame retries

DRETRI	EQU	1	;number of device retries

CTIM	EQU	2	;command frame ACK timeout

NBUFSZ	EQU	40	;print normal buffer size

DBUFSZ	EQU	20	;print double buffer size

SBUFSZ	EQU	29	;print sideways buffer size

SUBTTL	'System Address Equates'

SPACE	4,10

**	Page Zero Address Equates

LNFLG	EQU	$0000	;1-byte LNBUG flag (0 = not LNBUG)

NGFLAG	EQU	$0001	;1-byte memory status (0 = failure)

;	Not Cleared

CASINI	EQU	$0002	;2-byte cassette program initialization address

RAMLO	EQU	$0004	;2-byte RAM address for memory test

TRAMSZ	EQU	$0006	;1-byte RAM size temporary

CMCMD	EQU	$0007	;1-byte command communications

;	Cleared upon Coldstart Only

WARMST	EQU	$0008	;1-byte warmstart flag (0 = coldstart)

BOOT?	EQU	$0009	;1-byte successful boot flags

DOSVEC	EQU	$000A	;2-byte disk program start vector

DOSINI	EQU	$000C	;2-byte disk program initialization address

APPMHI	EQU	$000E	;2-byte applications memory high limit

;	Cleared upon Coldstart or Warmstart

INTZBS	EQU	$0010	;first page zero location to clear

POKMSK	EQU	$0010	;1-byte IRQEN shadow

BRKKEY	EQU	$0011	;1-byte BREAK key flag (0 = no BREAK)

RTCLOK	EQU	$0012	;3-byte real time clock (16 millisecond units)

BUFADR	EQU	$0015	;2-byte disk interface buffer address

ICCOMT	EQU	$0017	;1-byte CIO command table index

DSKFMS	EQU	$0018	;2-byte DOS File Management System pointer

DSKUTL	EQU	$001A	;2-byte DOS utility pointer

ABUFPT	EQU	$001C	;4-byte ACMI buffer pointer area

ZIOCB	EQU	$0020	;address of page zero IOCB

IOCBAS	EQU	$0020	;16-byte page zero IOCB

ICHIDZ	EQU	$0020	;1-byte handler ID ($FF = IOCB free)

ICDNOZ	EQU	$0021	;1-byte device number

ICCOMZ	EQU	$0022	;1-byte command code

ICSTAZ	EQU	$0023	;1-byte status of last action

ICBALZ	EQU	$0024	;1-byte low buffer address

ICBAHZ	EQU	$0025	;1-byte high buffer address

ICPTLZ	EQU	$0026	;1-byte low PUT-BYTE routine address-1

ICPTHZ	EQU	$0027	;1-byte high PUT-BYTE routine address-1

ICBLLZ	EQU	$0028	;1-byte low buffer length

ICBLHZ	EQU	$0029	;1-byte high buffer length

ICAX1Z	EQU	$002A	;1-byte first auxiliary information

ICAX2Z	EQU	$002B	;1-byte second auxiliary information

ICSPRZ	EQU	$002C	;4-byte spares

ENTVEC	EQU	$002C	;2-byte (not used)

ICIDNO	EQU	$002E	;1-byte IOCB index (IOCB number times IOCBSZ)

CIOCHR	EQU	$002F	;1-byte character for current CIO operation

STATUS	EQU	$0030	;1-byte SIO operation status

CHKSUM	EQU	$0031	;1-byte checksum (single byte sum with carry)

BUFRLO	EQU	$0032	;1-byte low data buffer address

BUFRHI	EQU	$0033	;1-byte high data buffer address

BFENLO	EQU	$0034	;1-byte low data buffer end address

BFENHI	EQU	$0035	;1-byte high data buffer end address

LTEMP	EQU	$0036	;2-byte relocating loader temporary

BUFRFL	EQU	$0038	;1-byte data buffer full flag (0 = not full)

RECVDN	EQU	$0039	;1-byte receive-frame done flag (0 = not done)

XMTDON	EQU	$003A	;1-byte transmit-frame done flag (0 = not done)

CHKSNT	EQU	$003B	;1-byte checksum sent flag (0 = not sent)

NOCKSM	EQU	$003C	;1-byte no checksum follows data flag (0 = does)

BPTR	EQU	$003D	;1-byte cassette buffer pointer

FTYPE	EQU	$003E	;1-byte cassette IRG type (neg. = continuous)

FEOF	EQU	$003F	;1-byte cassette EOF flag (0 = no EOF)

FREQ	EQU	$0040	;1-byte cassette beep counter

SOUNDR	EQU	$0041	;1-byte noisy I/O flag (0 = quiet)

CRITIC	EQU	$0042	;1-byte critical section flag (0 = not critical)

FMSZPG	EQU	$0043	;7-byte reserved for DOS File Management System

ZCHAIN	EQU	$004A	;2-byte handler linkage chain pointer

DSTAT	EQU	$004C	;1-byte display status

ATRACT	EQU	$004D	;1-byte attract-mode timer and flag

DRKMSK	EQU	$004E	;1-byte attract-mode dark (luminance) mask

COLRSH	EQU	$004F	;1-byte attract-mode color shift

TMPCHR	EQU	$0050	;1-byte	temporary character

HOLD1	EQU	$0051	;1-byte	temporary

LMARGN	EQU	$0052	;1-byte text column left margin

RMARGN	EQU	$0053	;1-byte text column right margin

ROWCRS	EQU	$0054	;1-byte cursor row

COLCRS	EQU	$0055	;2-byte cursor column

DINDEX	EQU	$0057	;1-byte display mode

SAVMSC	EQU	$0058	;2-byte saved memory scan counter

OLDROW	EQU	$005A	;1-byte prior row

OLDCOL	EQU	$005B	;2-byte prior column

OLDCHR	EQU	$005D	;1-byte saved character under cursor

OLDADR	EQU	$005E	;2-byte saved cursor memory address

FKDEF	EQU	$0060	;2-byte function key definition table address

PALNTS	EQU	$0062	;1-byte PAL/NTSC indicator (0 = NTSC, PAL = 1)

LOGCOL	EQU	$0063	;1-byte logical line cursor column

ADRESS	EQU	$0064	;2-byte temporary address

MLTTMP	EQU	$0066	;1-byte temporary

OPNTMP	EQU	$0066	;1-byte open temporary

TOADR	EQU	$0066	;2-byte destination address

SAVADR	EQU	$0068	;2-byte saved address

FRMADR	EQU	$0068	;2-byte source address

RAMTOP	EQU	$006A	;1-byte RAM size

BUFCNT	EQU	$006B	;1-byte buffer count (logical line size)

BUFSTR	EQU	$006C	;2-byte buffer start pointer

BITMSK	EQU	$006E	;1-byte bit mask for bit map operation

SHFAMT	EQU	$006F	;1-byte shift amount for pixel justification

ROWAC	EQU	$0070	;2-byte draw working row

COLAC	EQU	$0072	;2-byte draw working column

ENDPT	EQU	$0074	;2-byte end point

DELTAR	EQU	$0076	;1-byte row difference

DELTAC	EQU	$0077	;2-byte column difference

KEYDEF	EQU	$0079	;2-byte key definition table address

SWPFLG	EQU	$007B	;1-byte split screen swap flag (0 = not swapped)

HOLDCH	EQU	$007C	;1-byte temporary character

INSDAT	EQU	$007D	;1-byte temporary

COUNTR	EQU	$007E	;2-byte draw iteration count

;	Reserved for Application and Floating Point Package

;	EQU	$0080	;128 bytes reserved for application and FPP

SPACE	4,10

**	Floating Point Package Page Zero Address Equates

FR0	EQU	$00D4	;6-byte register 0

FR0M	EQU	$00D5	;5-byte register 0 mantissa

QTEMP	EQU	$00D9	;1-byte temporary

FRE	EQU	$00DA	;6-byte (internal) register E

FR1	EQU	$00E0	;6-byte register 1

FR1M	EQU	$00E1	;5-byte register 1 mantissa

FR2	EQU	$00E6	;6-byte (internal) register 2

FRX	EQU	$00EC	;1-byte temporary

EEXP	EQU	$00ED	;1-byte value of exponent

FRSIGN	EQU	$00EE	;1-byte floating point sign

NSIGN	EQU	$00EE	;1-byte sign of number

PLYCNT	EQU	$00EF	;1-byte polynomial degree

ESIGN	EQU	$00EF	;1-byte sign of exponent

SGNFLG	EQU	$00F0	;1-byte sign flag

FCHFLG	EQU	$00F0	;1-byte first character flag

XFMFLG	EQU	$00F1	;1-byte transform flag

DIGRT	EQU	$00F1	;1-byte number of digits after decimal point

CIX	EQU	$00F2	;1-byte current input index

INBUFF	EQU	$00F3	;2-byte line input buffer

ZTEMP1	EQU	$00F5	;2-byte temporary

ZTEMP4	EQU	$00F7	;2-byte temporary

ZTEMP3	EQU	$00F9	;2-byte temporary

FLPTR	EQU	$00FC	;2-byte floating point number pointer

FPTR2	EQU	$00FE	;2-byte floating point number pointer

SPACE	4,10

**	Page One (Stack) Address Equates

;	EQU	$0100	;256-byte stack

SPACE	4,10

**	Page Two Address Equates

INTABS	EQU	$0200	;42-byte interrupt handler table

VDSLST	EQU	$0200	;2-byte display list NMI vector

VPRCED	EQU	$0202	;2-byte serial I/O proceed line IRQ vector

VINTER	EQU	$0204	;2-byte serial I/O interrupt line IRQ vector

VBREAK	EQU	$0206	;2-byte BRK instruction IRQ vector

VKEYBD	EQU	$0208	;2-byte keyboard IRQ vector

VSERIN	EQU	$020A	;2-byte serial input ready IRQ vector

VSEROR	EQU	$020C	;2-byte serial output ready IRQ vector

VSEROC	EQU	$020E	;2-byte serial output complete IRQ vector

VTIMR1	EQU	$0210	;2-byte POKEY timer 1 IRQ vector

VTIMR2	EQU	$0212	;2-byte POKEY timer 2 IRQ vector

VTIMR4	EQU	$0214	;2-byte POKEY timer 4 IRQ vector

VIMIRQ	EQU	$0216	;2-byte immediate IRQ vector

CDTMV1	EQU	$0218	;2-byte countdown timer 1 value

CDTMV2	EQU	$021A	;2-byte countdown timer 2 value

CDTMV3	EQU	$021C	;2-byte countdown timer 3 value

CDTMV4	EQU	$021E	;2-byte countdown timer 4 value

CDTMV5	EQU	$0220	;2-byte countdown timer 5 value

VVBLKI	EQU	$0222	;2-byte immediate VBLANK NMI vector

VVBLKD	EQU	$0224	;2-byte deferred VBLANK NMI vector

CDTMA1	EQU	$0226	;2-byte countdown timer 1 vector

CDTMA2	EQU	$0228	;2-byte countdown timer 2 vector

CDTMF3	EQU	$022A	;1-byte countdown timer 3 flag (0 = expired)

SRTIMR	EQU	$022B	;1-byte software key repeat timer

CDTMF4	EQU	$022C	;1-byte countdown timer 4 flag (0 = expired)

INTEMP	EQU	$022D	;1-byte temporary

CDTMF5	EQU	$022E	;1-byte countdown timer 5 flag (0 = expired)

SDMCTL	EQU	$022F	;1-byte DMACTL shadow

SDLSTL	EQU	$0230	;1-byte DLISTL shadow

SDLSTH	EQU	$0231	;1-byte DLISTH shadow

SSKCTL	EQU	$0232	;1-byte SKCTL shadow

LCOUNT	EQU	$0233	;1-byte relocating loader record length counter

LPENH	EQU	$0234	;1-byte light pen horizontal value

LPENV	EQU	$0235	;1-byte light pen vertical value

BRKKY	EQU	$0236	;2-byte BREAK key vector

VPIRQ	EQU	$0238	;2-byte parallel device IRQ vector

CDEVIC	EQU	$023A	;1-byte command frame device ID

CCOMND	EQU	$023B	;1-byte command frame command

CAUX1	EQU	$023C	;1-byte command auxiliary 1

CAUX2	EQU	$023D	;1-byte command auxiliary 2

TEMP	EQU	$023E	;1-byte temporary

ASSERT	low TEMP<>$FF	;may not be the last word on a page

ERRFLG	EQU	$023F	;1-byte I/O error flag (0 = no error)

ASSERT	low ERRFLG<>$FF	;may not be the last word on a page

DFLAGS	EQU	$0240	;1-byte disk flags from sector 1

DBSECT	EQU	$0241	;1-byte disk boot sector count

BOOTAD	EQU	$0242	;2-byte disk boot memory address

COLDST	EQU	$0244	;1-byte coldstart flag (0 = complete)

RECLEN	EQU	$0245	;1-byte relocating loader record length

DSKTIM	EQU	$0246	;1-byte disk format timeout

PDVMSK	EQU	$0247	;1-byte parallel device selection mask

SHPDVS	EQU	$0248	;1-byte PDVS (parallel device selection) shadow

PDIMSK	EQU	$0249	;1-byte parallel device IRQ selection mask

RELADR	EQU	$024A	;2-byte relocating loader relative address

PPTMPA	EQU	$024C	;1-byte parallel device handler temporary

PPTMPX	EQU	$024D	;1-byte parallel device handler temporary

;	EQU	$024E	;6 bytes reserved for Atari

;	EQU	$0254	;23 bytes reserved for Atari

CHSALT	EQU	$026B	;1-byte character set alternate

VSFLAG	EQU	$026C	;1-byte fine vertical scroll count

KEYDIS	EQU	$026D	;1-byte keyboard disable

FINE	EQU	$026E	;1-byte fine scrolling mode

GPRIOR	EQU	$026F	;1-byte PRIOR shadow

PADDL0	EQU	$0270	;1-byte potentiometer 0

PADDL1	EQU	$0271	;1-byte potentiometer 1

PADDL2	EQU	$0272	;1-byte potentiometer 2

PADDL3	EQU	$0273	;1-byte potentiometer 3

PADDL4	EQU	$0274	;1-byte potentiometer 4

PADDL5	EQU	$0275	;1-byte potentiometer 5

PADDL6	EQU	$0276	;1-byte potentiometer 6

PADDL7	EQU	$0277	;1-byte potentiometer 7

STICK0	EQU	$0278	;1-byte joystick 0

STICK1	EQU	$0279	;1-byte joystick 1

STICK2	EQU	$027A	;1-byte joystick 2

STICK3	EQU	$027B	;1-byte joystick 3

PTRIG0	EQU	$027C	;1-byte paddle trigger 0

PTRIG1	EQU	$027D	;1-byte paddle trigger 1

PTRIG2	EQU	$027E	;1-byte paddle trigger 2

PTRIG3	EQU	$027F	;1-byte paddle trigger 3

PTRIG4	EQU	$0280	;1-byte paddle trigger 4

PTRIG5	EQU	$0281	;1-byte paddle trigger 5

PTRIG6	EQU	$0282	;1-byte paddle trigger 6

PTRIG7	EQU	$0283	;1-byte paddle trigger 7

STRIG0	EQU	$0284	;1-byte joystick trigger 0

STRIG1	EQU	$0285	;1-byte joystick trigger 1

STRIG2	EQU	$0286	;1-byte joystick trigger 2

STRIG3	EQU	$0287	;1-byte joystick trigger 3

HIBYTE	EQU	$0288	;1-byte relocating loader high byte value

WMODE	EQU	$0289	;1-byte cassette WRITE mode ($80 = writing)

BLIM	EQU	$028A	;1-byte cassette buffer limit

IMASK	EQU	$028B	;1-byte (not used)

JVECK	EQU	$028C	;2-byte jump vector or temporary

NEWADR	EQU	$028E	;2-byte relocating address

TXTROW	EQU	$0290	;1-byte split screen text cursor row

TXTCOL	EQU	$0291	;2-byte split screen text cursor column

TINDEX	EQU	$0293	;1-byte split scree text mode

TXTMSC	EQU	$0294	;2-byte split screen memory scan counter

TXTOLD	EQU	$0296	;6-byte OLDROW, OLDCOL, OLDCHR, OLDADR for text

CRETRY	EQU	$029C	;1-byte number of command frame retries

HOLD3	EQU	$029D	;1-byte temporary

SUBTMP	EQU	$029E	;1-byte temporary

HOLD2	EQU	$029F	;1-byte (not used)

DMASK	EQU	$02A0	;1-byte display (pixel location) mask

TMPLBT	EQU	$02A1	;1-byte (not used)

ESCFLG	EQU	$02A2	;1-byte escape flag ($80 = ESC detected)

** 	TABMAP	EQU	$02A3	;15-byte (120-bit) tab stop bit map

INVFLG	EQU	$02B6	;1-byte inverse video flag ($80 = inverse)

FILFLG	EQU	$02B7	;1-byte right fill flag (0 = no fill)

TMPROW	EQU	$02B8	;1-byte temporary row

TMPCOL	EQU	$02B9	;2-byte temporary column

SCRFLG	EQU	$02BB	;1-byte scroll occurence flag (0 = not occurred)

HOLD4	EQU	$02BC	;1-byte temporary

DRETRY	EQU	$02BD	;1-byte number of device retries

SHFLOK	EQU	$02BE	;1-byte shift/control lock flags

BOTSCR	EQU	$02BF	;1-byte screen bottom (24 = normal, 4 = split)

PCOLR0	EQU	$02C0	;1-byte player-missle 0 color/luminance

PCOLR1	EQU	$02C1	;1-byte player-missle 1 color/luminance

PCOLR2	EQU	$02C2	;1-byte player-missle 2 color/luminance

PCOLR3	EQU	$02C3	;1-byte player-missle 3 color/luminance

COLOR0	EQU	$02C4	;1-byte playfield 0 color/luminance

COLOR1	EQU	$02C5	;1-byte playfield 1 color/luminance

COLOR2	EQU	$02C6	;1-byte playfield 2 color/luminance

COLOR3	EQU	$02C7	;1-byte playfield 3 color/luminance

COLOR4	EQU	$02C8	;1-byte background color/luminance

PARMBL	EQU	$02C9	;6-byte relocating loader parameter block

RUNADR	EQU	$02C9	;2-byte run address

HIUSED	EQU	$02CB	;2-byte highest non-zero page address

ZHIUSE	EQU	$02CD	;2-byte highest zero page address

OLDPAR	EQU	$02CF	;6-byte relocating loader parameter block

GBYTEA	EQU	$02CF	;2-byte GET-BYTE routine address

LOADAD	EQU	$02D1	;2-byte non-zero page load address

ZLOADA	EQU	$02D3	;2-byte zero page load address

DSCTLN	EQU	$02D5	;2-byte disk sector length

ACMISR	EQU	$02D7	;2-byte ACMI interrupt service routine address

KRPDEL	EQU	$02D9	;1-byte auto-repeat delay

KEYREP	EQU	$02DA	;1-byte auto-repeat rate

NOCLIK	EQU	$02DB	;1-byte key click disable

HELPFG	EQU	$02DC	;1-byte HELP key flag (0 = no HELP)

DMASAV	EQU	$02DD	;1-byte SDMCTL save/restore

PBPNT	EQU	$02DE	;1-byte printer buffer pointer

PBUFSZ	EQU	$02DF	;1-byte printer buffer size

;	EQU	$02E0	;4 bytes reserved for DOS

RAMSIZ	EQU	$02E4	;1-byte high RAM size

MEMTOP	EQU	$02E5	;2-byte top of available user memory

MEMLO	EQU	$02E7	;2-byte bottom of available user memory

HNDLOD	EQU	$02E9	;1-byte user load flag (0 = no handler load)

DVSTAT	EQU	$02EA	;4-byte device status buffer

CBAUDL	EQU	$02EE	;1-byte low cassette baud rate

CBAUDH	EQU	$02EF	;1-byte high cassette baud rate

CRSINH	EQU	$02F0	;1-byte cursor inhibit (0 = cursor on)

KEYDEL	EQU	$02F1	;1-byte key debounce delay timer

CH1	EQU	$02F2	;1-byte prior keyboard character

CHACT	EQU	$02F3	;1-byte CHACTL shadow

CHBAS	EQU	$02F4	;1-byte CHBASE shadow

NEWROW	EQU	$02F5	;1-byte draw destination row

NEWCOL	EQU	$02F6	;2-byte draw destination column

ROWINC	EQU	$02F8	;1-byte draw row increment

COLINC	EQU	$02F9	;1-byte draw column increment

CHAR	EQU	$02FA	;1-byte internal character

ATACHR	EQU	$02FB	;1-byte ATASCII character or plot point

CH	EQU	$02FC	;1-byte keyboard code (buffer)

FILDAT	EQU	$02FD	;1-byte right fill data

DSPFLG	EQU	$02FE	;1-byte control character display flag (0 = no)

SSFLAG	EQU	$02FF	;1-byte start/stop flag (0 = not stopped)

SPACE	4,10

**	Page Three Address Equates

DCB	EQU	$0300	;12-byte device control block

DDEVIC	EQU	$0300	;1-byte unit 1 bus ID

DUNIT	EQU	$0301	;1-byte unit number

DCOMND	EQU	$0302	;1-byte bus command

DSTATS	EQU	$0303	;1-byte command type/status return

DBUFLO	EQU	$0304	;1-byte low data buffer address

DBUFHI	EQU	$0305	;1-byte high data buffer address

DTIMLO	EQU	$0306	;1-byte timeout (seconds)

DUNUSE	EQU	$0307	;1-byte (not used)

DBYTLO	EQU	$0308	;1-byte low number of bytes to transfer

DBYTHI	EQU	$0309	;1-byte high number of bytes to transfer

DAUX1	EQU	$030A	;1-byte first command auxiliary

DAUX2	EQU	$030B	;1-byte second command auxiliary

TIMER1	EQU	$030C	;2-byte initial baud rate timer value

JMPERS	EQU	$030E	;1-byte jumper options

CASFLG	EQU	$030F	;1-byte cassette I/O flag (0 = not cassette I/O)

TIMER2	EQU	$0310	;2-byte final baud rate timer value

TEMP1	EQU	$0312	;2-byte temporary

TEMP2	EQU	$0313	;1-byte temporary

PTIMOT	EQU	$0314	;1-byte printer timeout

TEMP3	EQU	$0315	;1-byte temporary

SAVIO	EQU	$0316	;1-byte saved serial data input indicator

TIMFLG	EQU	$0317	;1-byte timeout flag (0 = timeout)

STACKP	EQU	$0318	;1-byte SIO saved stack pointer

TSTAT	EQU	$0319	;1-byte temporary status

HATABS	EQU	$031A	;35-byte handler address table

PUPBT1	EQU	$033D	;1-byte power-up validation byte 1

PUPBT2	EQU	$033E	;1-byte power-up validation byte 2

PUPBT3	EQU	$033F	;1-byte power-up validation byte 3

IOCB	EQU	$0340	;128-byte I/O control blocks area

ICHID	EQU	$0340	;1-byte handler ID ($FF = free)

ICDNO	EQU	$0341	;1-byte device number

ICCOM	EQU	$0342	;1-byte command code

ICSTA	EQU	$0343	;1-byte status of last action

ICBAL	EQU	$0344	;1-byte low buffer address

ICBAH	EQU	$0345	;1-byte high buffer address

ICPTL	EQU	$0346	;1-byte low PUT-BYTE routine address-1

ICPTH	EQU	$0347	;1-byte high PUT-BYTE routine address-1

ICBLL	EQU	$0348	;1-byte low buffer length

ICBLH	EQU	$0349	;1-byte high buffer length

ICAX1	EQU	$034A	;1-byte first auxiliary information

ICAX2	EQU	$034B	;1-byte second auxiliary information

ICSPR	EQU	$034C	;4-byte work area

PRNBUF	EQU	$03C0	;40-byte printer buffer

SUPERF	EQU	$03E8	;1-byte editor super function flag (0 = not)

CKEY	EQU	$03E9	;1-byte cassette boot request flag (0 = not)

CASSBT	EQU	$03EA	;1-byte cassette boot flag (0 = not)

CARTCK	EQU	$03EB	;1-byte cartridge equivalence checksum

DERRF	EQU	$03EC	;1-byte screen OPEN error flag (0 = not)

;	Remainder of Page Three Not Cleared upon Reset

ACMVAR	EQU	$03ED	;11 bytes reserved for ACMI

BASICF	EQU	$03F8	;1-byte BASIC switch flag (0 = BASIC enabled)

MINTLK	EQU	$03F9	;1-byte ACMI module interlock

GINTLK	EQU	$03FA	;1-byte cartridge interlock

CHLINK	EQU	$03FB	;2-byte loaded handler chain link

CASBUF	EQU	$03FD	;3-byte first 3 bytes of cassette buffer

SPACE	4,10

**	Page Four Address Equates

;	EQU	$0400	;128-byte remainder of cassette buffer

;	Reserved for Application

USAREA	EQU	$0480	;128 bytes reserved for application

SPACE	4,10

**	Page Five Address Equates

;	Reserved for Application and Floating Point Package

;	EQU	$0500	;256 bytes reserved for application and FPP

SPACE	4,10

**	Floating Point Package Address Equates

LBPR1	EQU	$057E	;1-byte LBUFF preamble

LBPR2	EQU	$057F	;1-byte LBUFF preamble

LBUFF	EQU	$0580	;128-byte line buffer

PLYARG	EQU	$05E0	;6-byte floating point polynomial argument

FPSCR	EQU	$05E6	;6-byte floating point temporary

FPSCR1	EQU	$05EC	;6-byte floating point temporary

SPACE	4,10

**	Page Six Address Equates

;	Reserved for Application

;	EQU	$0600	;256 bytes reserved for application

SPACE	4,10

**	LNBUG Address Equates

LNBUG	IF	LNBUG

LNORG	EQU	$6000	;LNBUG origin

LNIRQ	EQU	$6033	;LNBUG IRQ entry

LNNMI	EQU	$8351	;LNBUG NMI vector

LNBUG	ENDIF

SPACE	4,10

**	Cartridge Address Equates

CARTCS	EQU	$BFFA	;2-byte cartridge coldstart address

CART	EQU	$BFFC	;1-byte cartridge present indicator

CARTFG	EQU	$BFFD	;1-byte cartridge flags

CARTAD	EQU	$BFFE	;2-byte cartridge start vector

SPACE	4,10

**	CTIA/GTIA Address Equates

CTIA	EQU	$D000	;CTIA/GTIA area

;	Read/Write Addresses

CONSOL	EQU	$D01F	;console switches and speaker control

;	Read Addresses

M0PF	EQU	$D000	;missle 0 and playfield collision

M1PF	EQU	$D001	;missle 1 and playfield collision

M2PF	EQU	$D002	;missle 2 and playfield collision

M3PF	EQU	$D003	;missle 3 and playfield collision

P0PF	EQU	$D004	;player 0 and playfield collision

P1PF	EQU	$D005	;player 1 and playfield collision

P2PF	EQU	$D006	;player 2 and playfield collision

P3PF	EQU	$D007	;player 3 and playfield collision

M0PL	EQU	$D008	;missle 0 and player collision

M1PL	EQU	$D009	;missle 1 and player collision

M2PL	EQU	$D00A	;missle 2 and player collision

M3PL	EQU	$D00B	;missle 3 and player collision

P0PL	EQU	$D00C	;player 0 and player collision

P1PL	EQU	$D00D	;player 1 and player collision

P2PL	EQU	$D00E	;player 2 and player collision

P3PL	EQU	$D00F	;player 3 and player collision

TRIG0	EQU	$D010	;joystick trigger 0

TRIG1	EQU	$D011	;joystick trigger 1

TRIG2	EQU	$D012	;cartridge interlock

TRIG3	EQU	$D013	;ACMI module interlock

PAL	EQU	$D014	;PAL/NTSC indicator

;	Write Addresses

HPOSP0	EQU	$D000	;player 0 horizontal position

HPOSP1	EQU	$D001	;player 1 horizontal position

HPOSP2	EQU	$D002	;player 2 horizontal position

HPOSP3	EQU	$D003	;player 3 horizontal position

HPOSM0	EQU	$D004	;missle 0 horizontal position

HPOSM1	EQU	$D005	;missle 1 horizontal position

HPOSM2	EQU	$D006	;missle 2 horizontal position

HPOSM3	EQU	$D007	;missle 3 horizontal position

SIZEP0	EQU	$D008	;player 0 size

SIZEP1	EQU	$D009	;player 1 size

SIZEP2	EQU	$D00A	;player 2 size

SIZEP3	EQU	$D00B	;player 3 size

SIZEM	EQU	$D00C	;missle sizes

GRAFP0	EQU	$D00D	;player 0 graphics

GRAFP1	EQU	$D00E	;player 1 graphics

GRAFP2	EQU	$D00F	;player 2 graphics

GRAFP3	EQU	$D010	;player 3 graphics

GRAFM	EQU	$D011	;missle graphics

COLPM0	EQU	$D012	;player-missle 0 color/luminance

COLPM1	EQU	$D013	;player-missle 1 color/luminance

COLPM2	EQU	$D014	;player-missle 2 color/luminance

COLPM3	EQU	$D015	;player-missle 3 color/luminance

COLPF0	EQU	$D016	;playfield 0 color/luminance

COLPF1	EQU	$D017	;playfield 1 color/luminance

COLPF2	EQU	$D018	;playfield 2 color/luminance

COLPF3	EQU	$D019	;playfield 3 color/luminance

COLBK	EQU	$D01A	;background color/luminance

PRIOR	EQU	$D01B	;priority select

VDELAY	EQU	$D01C	;vertical delay

GRACTL	EQU	$D01D	;graphic control

HITCLR	EQU	$D01E	;collision clear

SPACE	4,10

**	PBI Address Equates

PBI	EQU	$D100	;parallel bus interface area

;	Read Addresses

PDVI	EQU	$D1FF	;parallel device IRQ status

;	Write Addresses

PDVS	EQU	$D1FF	;parallel device select

SPACE	4,10

**	POKEY Address Equates

POKEY	EQU	$D200	;POKEY area

;	Read Addresses

POT0	EQU	$D200	;potentiometer 0

POT1	EQU	$D201	;potentiometer 1

POT2	EQU	$D202	;potentiometer 2

POT3	EQU	$D203	;potentiometer 3

POT4	EQU	$D204	;potentiometer 4

POT5	EQU	$D205	;potentiometer 5

POT6	EQU	$D206	;potentiometer 6

POT7	EQU	$D207	;potentiometer 7

ALLPOT	EQU	$D208	;potentiometer port state

KBCODE	EQU	$D209	;keyboard code

RANDOM	EQU	$D20A	;random number generator

SERIN	EQU	$D20D	;serial port input

IRQST	EQU	$D20E	;IRQ interrupt status

SKSTAT	EQU	$D20F	;serial port and keyboard status

;	Write Addresses

AUDF1	EQU	$D200	;channel 1 audio frequency

AUDC1	EQU	$D201	;channel 1 audio control

AUDF2	EQU	$D202	;channel 2 audio frequency

AUDC2	EQU	$D203	;channel 2 audio control

AUDF3	EQU	$D204	;channel 3 audio frequency

AUDC3	EQU	$D205	;channel 3 audio control

AUDF4	EQU	$D206	;channel 4 audio frequency

AUDC4	EQU	$D207	;channel 4 audio control

AUDCTL	EQU	$D208	;audio control

STIMER	EQU	$D209	;start timers

SKRES	EQU	$D20A	;reset SKSTAT status

POTGO	EQU	$D20B	;start potentiometer scan sequence

SEROUT	EQU	$D20D	;serial port output

IRQEN	EQU	$D20E	;IRQ interrupt enable

SKCTL	EQU	$D20F	;serial port and keyboard control

SPACE	4,10

**	PIA Address Equates

PIA	EQU	$D300	;PIA area

;	Read/Write Addresses

PORTA	EQU	$D300	;port A direction register or jacks 0 and 1

PORTB	EQU	$D301	;port B direction register or memory control

PACTL	EQU	$D302	;port A control

PBCTL	EQU	$D303	;port B control

SPACE	4,10

**	ANTIC Address Equates

ANTIC	EQU	$D400	;ANTIC area

;	Read Addresses

VCOUNT	EQU	$D40B	;vertical line counter

PENH	EQU	$D40C	;light pen horizontal position

PENV	EQU	$D40D	;light pen vertical position

NMIST	EQU	$D40F	;NMI interrupt status

;	Write Addresses

DMACTL	EQU	$D400	;DMA control

CHACTL	EQU	$D401	;character control

DLISTL	EQU	$D402	;low display list address

DLISTH	EQU	$D403	;high disply list address

HSCROL	EQU	$D404	;horizontal scroll

VSCROL	EQU	$D405	;vertical scroll

PMBASE	EQU	$D407	;player-missle base address

CHBASE	EQU	$D409	;character base address

WSYNC	EQU	$D40A	;wait for HBLANK synchronization

NMIEN	EQU	$D40E	;NMI enable

NMIRES	EQU	$D40F	;NMI interrupt status reset

SPACE	4,10

**	PBI RAM Address Equates

PBIRAM	EQU	$D600	;parallel bus interface RAM area

SPACE	4,10

**	ACMI Address Equates

ACMI	IF	ACMI

AMVTAD	EQU	$D7EA	;vector table

AMINIT	EQU	$D7F6	;initialization address

AMNAME	EQU	$D7F9	;ACMI device code

AMISRA	EQU	$D7FA	;interrupt service routine address

AMFLAG	EQU	$D7FF	;ACMI flags

ACMI	ENDIF

SPACE	4,10

**	Floating Point Package Address Equates

*	deleted

SPACE	4,10

**	Parallel Device Address Equates

PDID1	EQU	$D803	;parallel device ID 1

PDIOV	EQU	$D805	;parallel device I/O vector

PDIRQV	EQU	$D808	;parallel device IRQ vector

PDID2	EQU	$D80B	;parallel device ID 2

PDVV	EQU	$D80D	;parallel device vector table

SPACE	4,10

**	Device Handler Vector Table Address Equates

* Redefined in 6545 Handler

* EDITRV	EQU	$E400	;editor handler vector table

* SCRENV	EQU	$E410	;screen handler vector table

* KEYBDV	EQU	$E420	;keyboard handler vector table

PRINTV	EQU	$E430	;printer handler vector table

CASETV	EQU	$E440	;cassette handler vector table

SPACE	4,10

**	Jump Vector Address Equates

DINITV	EQU	$E450	;vector to initialize DIO

DSKINV	EQU	$E453	;vector to DIO

CIOV	EQU	$E456	;vector to CIO

SIOV	EQU	$E459	;vector to SIO

SETVBV	EQU	$E45C	;vector to set VBLANK parameters

SYSVBV	EQU	$E45F	;vector to process immediate VBLANK NMI

XITVBV	EQU	$E462	;vector to process deferred VBLANK NMI

SIOINV	EQU	$E465	;vector to initialize SIO

SENDEV	EQU	$E468	;vector to enable SEND

INTINV	EQU	$E46B	;vector to initialize interrupt handler

CIOINV	EQU	$E46E	;vector to initialize CIO

BLKBDV	EQU	$E471	;vector to power-up display (formerly memo pad)

WARMSV	EQU	$E474	;vector to warmstart

COLDSV	EQU	$E477	;vector to coldstart

RBLOKV	EQU	$E47A	;vector to read cassette block

CSOPIV	EQU	$E47D	;vector to open cassette for input

PUPDIV	EQU	$E480	;vector to power-up display

SLFTSV	EQU	$E483	;vector to self-test

PHENTV	EQU	$E486	;vector to enter peripheral handler

PHUNLV	EQU	$E489	;vector to unlink peripheral handler

PHINIV	EQU	$E48C	;vector to initialize peripheral handler

SPACE	4,10

**	Generic Parallel Device Handler Vector Table Address Equates

GPDVV	EQU	$E48F	;generic parallel device handler vector table

SUBTTL	'Miscellaneous Address Equates'

SPACE	4,10

**	Self-test Page Zero Address Equates

STTIME	EQU	$0080	;2-byte main screen timeout timer

STAUT	EQU	$0082	;1-byte auto-mode flag

STJMP	EQU	$0083	;3-byte ANTIC jump instruction

STSEL	EQU	$0086	;1-byte selection

STPASS	EQU	$0087	;1-byte pass

STSPP	EQU	$0088	;1-byte SELECT previously pressed flag

;	EQU	$0089	;1-byte (not used)

STKST	EQU	$008A	;1-byte keyboard self-test flag (0 = not)

STCHK	EQU	$008B	;2-byte checksum

STSMM	EQU	$008D	;1-byte screen memory mask

STSMP	EQU	$008E	;1-byte screen memory pointer

ST1K	EQU	$008F	;1-byte current 1K of memory to test

STPAG	EQU	$0090	;2-byte current page to test

STPC	EQU	$0092	;1-byte page count

STMVAL	EQU	$0093	;1-byte correct value for memory test

STSKP	EQU	$0094	;1-byte simulated keypress index

STTMP1	EQU	$0095	;2-byte temporary

STVOC	EQU	$0097	;1-byte current voice indicator

STNOT	EQU	$0098	;1-byte current note counter

STCDI	EQU	$0099	;1-byte cleft display pointer

STCDA	EQU	$009A	;1-byte cleft data pointer

STTMP2	EQU	$009B	;2-byte temporary

STTMP3	EQU	$009D	;1-byte temporary

STADR1	EQU	$009E	;2-byte temporary address

STADR2	EQU	$00A0	;2-byte temporary address

STBL	EQU	$00A2	;1-byte blink counter

STTMP4	EQU	$00A3	;1-byte temporary

STLM	EQU	$00A4	;1-byte LED mask

STTMP5	EQU	$00A5	;1-byte temporary

SPACE	4,10

**	Self-test Address Equates

ST3000	EQU	$3000	;screen memory

ST3002	EQU	$3002	;cleft display

ST3004	EQU	$3004	;"VOICE #" text display

ST300B	EQU	$300B	;voice number display

ST301C	EQU	$301C	;START key display

ST301E	EQU	$301E	;SELECT key display

ST3020	EQU	$3020	;OPTION key display, first 8K ROM display

ST3021	EQU	$3021	;keyboard character display

ST3022	EQU	$3022	;keyboard text display

ST3024	EQU	$3024	;second 8K ROM display

ST3028	EQU	$3028	;"RAM" text display

ST3038	EQU	$3038	;RAM display

ST303C	EQU	$303C	;fifth note display

ST304C	EQU	$304C	;"B S" text display

ST3052	EQU	$3052	;tab key display

ST3062	EQU	$3062	;cleft display

ST306D	EQU	$306D	;return key display

ST3072	EQU	$3072	;control key display

ST3092	EQU	$3092	;"SH" text display

ST309E	EQU	$309E	;sixth note display

ST30AB	EQU	$30AB	;"SH" text display

ST30B7	EQU	$30B7	;"S P A C E   B A R" text display

ST30C1	EQU	$30C1	;cleft display

ST30C2	EQU	$30C2	;cleft display

ST30C7	EQU	$30C7	;third note display

ST30CA	EQU	$30CA	;fourth note display

ST30F8	EQU	$30F8	;third note display

ST3100	EQU	$3100	;screen memory

ST3121	EQU	$3121	;cleft display

ST3122	EQU	$3122	;cleft display

ST313C	EQU	$313C	;fifth note display

ST3150	EQU	$3150	;first line of staff display

ST3154	EQU	$3154	;first note display

ST3181	EQU	$3181	;cleft display

ST3182	EQU	$3182	;cleft display

ST3186	EQU	$3186	;second note display

ST318C	EQU	$318C	;fifth note display

ST31B0	EQU	$31B0	;second line of staff display

ST31C2	EQU	$31C2	;cleft display

ST31CA	EQU	$31CA	;fourth note display

ST31EE	EQU	$31EE	;sixth note display

ST31F1	EQU	$31F1	;cleft display

ST3210	EQU	$3210	;third line of staff display

ST321A	EQU	$321A	;fourth note display

ST3248	EQU	$3248	;third note display

ST3270	EQU	$3270	;fourth line of staff display

ST32D0	EQU	$32D0	;fifth line of staff display

LIST	F,-M,L,R

SUBTTL	'6545 Handler Data Tables'

SPACE	4,10

**	PDROM	-	Parallel Device ROM Data Table

ORG	$D800	; This table must be at $D800.

DW	$0000	; (Optional) ROM Checksum (low byte, high byte)

DB	$00	; (Optional) Revision Number

DB	$80	; (Mandatory) ID Number 1

; Value = 80H

DB	$00	; (Optional) Name or Type

JMP	DHSV31-1 ; (Mandatory) Low-level I/O Vector

; Value = JMP address (low byte, high byte)

JMP	DHSV31-1 ; (Mandatory) IRQ Handler Vector

; Value = JMP address (low byte, high byte)

DB	$91	; (Mandatory) ID Number 2

; Value = 91H

DB	$00	; (Optional) Device Name

; Value = Device Name in ASCII

; 	Device Vector Table for K:, E: and S:

; 	Each non-Initialization command will call the Device Handler

; 	Selector routine.

;	PDVV = $D80D

DW	DHSV0-1 ; (Mandatory) Device Handler Open Vector

; Value = address-1 (low byte, high byte)

DW	DHSV0-1	; (Mandatory) Device Handler Close Vector

; Value = address-1 (low byte, high byte)

DW	DHSV0-1 ; (Mandatory) Device Handler Get-Byte Vector

; Value = address-1 (low byte, high byte)

DW	DHSV-1 	; (Mandatory) Device Handler Put-Byte Vector

; Value = address-1 (low byte, high byte)

DW	DHSV0-1 ; (Mandatory) Device Handler Status Vector

; Value = address-1 (low byte, high byte)

DW	DHSV0-1 ; (Mandatory) Device Handler Special Vector

; Value = address-1 (low byte, high byte)

JMP	PDSIN   ; (Mandatory) Initialization Vector

; Value = JMP address (low byte, high byte)

DW	$00	; (Optional) Not Used

; Value = 0

SPACE	4,10

**	CDT	-	6545 Controller Data Table

SPACE	4,10

; 6545 Register Data Table

; Reference: Synertek SY6545 Detailed System Implementations, Page 12.

SPACE	4,10

; -	Address Register

; -	Status Register

HTOT	DB	$71	; R0	Horizontal Total (N-1)

HDISP	DB	$50	; R1	Horizonatal Displayed

HSYNC	DB	$5A	; R2	Horizontal Sync Position

HVSW	DB	$0E	; R3	Horizontal and Vertical Sync Widths

VTOT	DB	$1F	; R4	Vertical Total (N-1)

VTADJ	DB	$06	; R5	Vertical Total Adjust

VDISP	DB	$19	; R6	Vertical Displayed

VSPOS	DB	$19	; R7	Vertical Sync Position

MODEC	DB	$00	; R8	Mode Control

SCANL   DB	$07	; R9	Scan Lines (N-1)

CURS	DB	$20	; R10	Cursor Start (changed from $00). Use a software cursor.

CURE	DB	$00	; R11	Cursor End (changed from $07)

START	DW	$0000	; R12	Start (Hi byte, Lo byte)

CUR	DW	$0000	; R13	Cursor (Hi byte, Lo byte)

LPEN	DW	$0000	; R14	LPEN (Hi byte, Lo byte)

UA	DW	$0000	; R15	Update Address (Hi byte, Lo byte)

; R31	Dummy Location

SPACE	4,10

**	6545 Controller Board Equates

AD6545	=	$D1FA	; 6545 Address

SA6545	=	$D1FC	; 6545 Screen Ram Address Latch

; Top 5 bits of data written into this register

; Select the address of the 2K screen buffer

EJECT

SPACE	4,10

**	List of Fixed OS Locations.	**

*

*	These equates specify locations in the 600xl/800xl operating

*	system rom that must be fixed in future os revisions in order

*	to function properly with this handler.

*

TCVO	=	$E72D	; Table of Command Vector Offsets

TAIC	=	$FB49   ; Table of ATASCII to Internal Conversion Constants

TIAC	=	$FB4D   ; Table of Internal to ATASCII Conversion Constants

TCKD	=	$FB51   ; Table of Character Key Definitions

TFKD	=	$FC11	; Table of Function Key Definitions

TBTM	=	$EEB4	; Table of Bit Masks

*SIN	=	$EF6E

*ESC	=	$F3E0

*ROD	=	$F718

*CIA	=	$F76A

*SRC	=	$F94C

*RRC	=	$F957

*SMS	=	$F9A6

KGB	=	$F2FD

IGN     =	$F2F8

SKC	=	$F983

CCC	=	$F93C

SPACE	4,10

**	Database Variables	**

*	(TEMPORARY FIX - ONLY WITH 800XL)

TABMAP	=	$9700	; 30 bytes * 8 bits = 240 characters.

LOGMAP	=	$971E   ;  4 Bytes

LOGADR	=	$9722	;  2 Bytes

EJECT

SPACE	4,10

**********************************************************

**	DHS - Device Handler Selector Vector		**

**********************************************************

*

*	Branches to the correct device vector table

*

*	Check the Device Type in the originating IOCB

*	 The x register contains the index to the originating IOCB.

*	 ICHID,X contains the index into HATABS

*	Branch to the corresponding handler vector

*	Set the C flag to indicate if the request was handled.

*	Entry - PPTMPA = A

*		PPTMPX = X

*	PROBLEM - BASIC CALLS PUT-BYTE DIRECTLY WITHOUT USING

*		  THE CIO INTERFACE.

;	Calculate command vector offset.

;	Note - Put-Byte has a special vector because Basic calls

;	the Put-byte routine without using CIO, and ICCOMT is not

;	initialized in that case.

DHSV    LDY	#$06		;Command Vector Offset for Put-Char ($0B) or Put-Record ($09) command.

LDA	#'E'		;Always the editor.

BNE	DHSV01		;always

DHSV0   LDY	ICCOMT		;NON PUT-BYTE COMMANDS

LDA	TCVO-3,Y	;Table of vector offsets for commands

TAY			;Table value-Offset for COMMAND VECTOR

LDX	ICHIDZ		;Get the Handler ID (index)

LDA	HATABS,X	;Device Name

DHSV01  CMP	#'E'

BNE	DHSV1

LDA	#HIGH [DHSV4-1]

PHA

LDA	#LOW  [DHSV4-1]

PHA

LDA	EDITRV+1,Y	;High routine address - 1

PHA

LDA	EDITRV,Y        ;Low routine address - 1

PHA

JMP	DHSV5	        ;Perform Command, Return to DHSV4

DHSV1	CMP	#'S'

BNE	DHSV2

LDA	#HIGH [DHSV4-1]

PHA

LDA	#LOW [DHSV4-1]

PHA

LDA	SCRENV+1,Y

PHA

LDA	SCRENV,Y

PHA

JMP	DHSV5	         ;Perform Command, Return to DHSV4

DHSV2	CMP	#'K'

BNE	DHSV3

LDA	#HIGH [DHSV4-1]

PHA

LDA	#LOW [DHSV4-1]

PHA

LDA	KEYBDV+1,Y

PHA

LDA	KEYBDV,Y

PHA

JMP	DHSV5		  ;Perform Command, Return to DHSV4

;	Request not handled.

;	Return to Generic Parallel Bus Device Handler.

DHSV3	PLA			;Discard the good exit address.

PLA

DHSV31  CLC			;Request not handled.

RTS     		;Return to Generic Parallel Device Handler.

;	Request handled.

;	Return to Generic Parallel Bus Device Handler.

DHSV4   SEC	                ;Request Handled.

RTS 			;Return to Generic Parallel Device Handler.

;	Restore the registers and call the command procedure

DHSV5	LDA	PPTMPA   	;A contains data for put-byte

LDX	PPTMPX          ;x contains iocb index

LDY	#$92            ;y contains #$92

RTS                     ;call the command procedure (address on stack)

EJECT

SPACE	4,10

**      Device Handler Vector Tables

SPACE	4,10

**	EDITRV - Editor Handler Vector Table

EDITRV

DW	EOP-1	;	perform editor OPEN

DW	SST-1	;os	perform editor CLOSE

DW	EGB-1	;	perform editor GET-BYTE

DW	EPB-1	;	perform editor PUT-BYTE

DW	SST-1	;os	perform editor STATUS (screen STATUS)

DW	ESP-1	;os	perform editor SPECIAL

;	JMP	PDSIN	;	initialize editor (initialize screen)

;	DB	0	;	reserved

SPACE	4,10

**	SCRENV - Screen Handler Vector Table

SCRENV

DW	SOP-1	;	perform screen OPEN

DW	SST-1	;os	perform screen CLOSE (editor CLOSE)

DW	SGB-1	;	perform screen GET-BYTE

DW	SPB-1	;	perform screen PUT-BYTE

DW	SST-1	;os	perform screen STATUS

DW	ESP-1	;os	perform screen SPECIAL

;	JMP	PDSIN	;	initialize screen

;	DB	0	;	reserved

SPACE	4,10

**	KEYBDV - Keyboard Handler Vector Table

KEYBDV

DW	SST-1	;os	perform keyboard OPEN (screen STATUS)

DW	SST-1	;os	perform keyboard CLOSE (screen STATUS)

DW	KGB-1	;os	perform keyboard GET-BYTE

DW	ESP-1	;os	perform keyboard SPECIAL (editor SPECIAL)

DW	SST-1	;os	perform keyboard STATUS (screen STATUS)

DW	ESP-1	;os	perform keyboard SPECIAL (editor SPECIAL)

;	JMP	PDSIN	;initialize keyboard (initialize screen)

;	DB	0	;reserved

SUBTTL	'Device Handler Commands'

SPACE	4,10

******************************************************************************************

**	PDSIN	- PARALLEL DEVICE SCREEN, EDITOR AND KEYBOARD INITIALIZATION COMMAND	**

******************************************************************************************

;	Call OS  Screen, Editor and Keyboard Initialization Vector

PDSIN	JSR	SIN

;	Establish right screen margin

LDA	#79

STA	RMARGN

;	Set PDVMSK

;	Load the 6545 device number n from the Parallel Device Shadow

;	Register and store this value in PDVMSK to indicate that

;	device number n exists on the parallel bus and to enable

;	Device handler requests or low-level IO requests.

LDA	SHPDVS

ORA	PDVMSK

STA	PDVMSK

;	Set PDIMSK

;	Used to enable Parallel Device IRQ processing.

;	Not Applicable.

;	Modify HATABS.

LDA	#'S'		;Display Device Name

JSR	SERCH

LDA	#'E'

JSR	SERCH

LDA	#'K'

JSR	SERCH

;	Return to the OS Initialization Procedure.

RTS

;	Search HATABS for Device Name and Insert Address of

;	the Generic Parallel Device Handler.

SERCH	LDY	#MAXDEV		;Offset to last possible entrty.

SERCH1	CMP	HATABS,Y

BEQ	SERCH3		;If device found.

DEY

DEY

DEY

BPL	SERCH1		;Check next entry.

;	Return to the generic parallel device handler with an error condition.

SERCH2	LDY	#NONDEV

PLA

PLA

RTS

;	Insert Generic Parallel Device Handler Address in Hatabs.

SERCH3	INY			;Offset to Handler Vector Table Entry Low Byte

LDA	#LOW GPDVV	;Insert low byte.

STA	HATABS,Y

INY             	;Offset to Handler Vector Table Entry High Byte

LDA	#HIGH GPDVV     ;Insert high byte.

STA	HATABS,Y

RTS

EJECT

SPACE	4,10

**	SIN - SCREEN, EDITOR AND KEYBOARD INITIALIZATION VECTOR

*

*	VEC	S: INIT VECTOR

*		E: INIT VECTOR

*		K: INIT VECTOR

*

*	ENTRY	JSR	SIN

*

*       LOC    	600XL/800XL OS ROM

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

*		2. 6545 Conversion

*		   R. L. Blotcky 4/26/84

SIN	=	*

LDA	#$FF		;clear code indicator

STA	CH		;key code

LDA	RAMSIZ		;size of RAM

STA	RAMTOP		;RAM size

LDA	#$40		;CAPS lock indicator

STA	SHFLOK		;shift/control lock flags

LDA	#low TCKD	;table of character key definitions

STA	KEYDEF		;key definition table address

LDA	#high TCKD

STA	KEYDEF+1

LDA	#low TFKD	;table of function key definitions

STA	FKDEF		;function key definition table address

LDA 	#high TFKD

STA	FKDEF+1

RTS			;return

EJECT

SPACE	4,10

******************************************

**	SOP - SCREEN OPEN COMMAND	**

******************************************

*

*	ENTRY	JSR	SOP

*

*	LOC	6545 Handler

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

*		2. 6545 Conversion

*		   Only Basic Mode 0 is supported.

*		   R. L. Blotcky 4/26/84

SOP	=	*	;entry

;	Check Mode.

LDA	ICAX2Z  ;error trap for non-zero modes

AND	#$0F

BNE	COC

;	Process Mode 0

;	JMP	EOP	;perform editor OPEN, return

EJECT

SPACE	4,10

**************************************************

**	EOP - Perform Editor OPEN COMMAND	**

**************************************************

*

*	ENTRY	JSR	EOP

*

*	LOC	6545 Handler

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

*		2. 6545 Conversion

*		   No change to the code. Control falls thru to COC.

*		   R. L. Blotcky 4/26/84

EOP	=	*	;entry

LDA	ICAX1Z

AND	#$0F

STA	ICAX1Z

LDA	#0

;	JMP	COC	;complete OPEN command, return

EJECT

SPACE	4,10

**	COC - Complete OPEN Command

*

*	ENTRY	JSR	COC

*		A = mode

*

*	LOC	6545 Handler

*

COC	=	*	;entry

;	1.0 Check mode.

;	If its not mode 0, then generate an error.

STA	DINDEX	;save mode

CMP	#$00	;

BEQ	COC1	;if mode 0

;	Error processing if not mode 0.

LDA	#BADMOD

JMP	COC17

;	2.0 Initialize for OPEN.

COC1

LDA	#0

STA	SDMCTL		;turn off DMA

LDA	#SUCCES

STA	DSTAT		;clear status

LDA	#$C0		;enable IRQ

ORA	POKMSK

STA	POKMSK

STA	IRQEN

;	Set NMIEN staus: disable DLI, enable vblank.

LDA	#$40		;disable DLI

STA	NMIEN

; 	Clear control.

LDA	#0

STA	ADRESS

STA	CRSINH

;	Set initial tab stops.

LDY	#29		;offset to last byte of bit map

LDA	#$01		;tab stop every 8 characters

COC3	STA	TABMAP,Y	;set tab stop

DEY

BPL	COC3		;if not done

;	3.0  Allocate 6545 Screen Ram, 2K = 8 pages.

LDA	RAMTOP		;(high) RAM size

SEC

SBC	#$08		;8 pages total

STA	ADRESS+1

;	Load the 6545 screen address register

STA	SA6545		;load 6545 screen address register

;	3.1 Temporary fix for variables from $D600

;	    (WORKS ONLY WITH AN 800XL)

DEC	RAMTOP

STA	ADRESS+1

;	Set memory scan counter.

;	Address of 1st byte of screen ram.

LDA	ADRESS

STA	SAVMSC		;save memory scan counter

LDA	ADRESS+1

STA	SAVMSC+1

;	4.0 Modify the Handler database variables.

LDA	#24  		;number of rows

STA	BOTSCR          ;this could be a DB statement.

LDA	#$FF

STA	ADRESS		;display list address

STA	MEMTOP		;update top of memory

DEC	ADRESS+1

LDA	ADRESS+1

STA	MEMTOP+1

;	Check status.

LDA	DSTAT		;status

BPL	COC18		;if no error

;	Process error.

COC17	STA	DERRF		;screen OPEN error flag

JSR	EOP		;perform editor OPEN

LDA	DERRF		;restore status

LDY	#0		;no screen OPEN error indicator

STY	DERRF		;screen OPEN error flag

TAY			;status

RTS			;return

;	Check clear inhibit.

COC18	LDA	ICAX1Z

AND	#$20		;extract clear inhibit bit

BNE	COC19		;if clear inhibited

;	Clear screen.

JSR	CSC		;clear screen

;	7.0 Setup the 6545 Hardware Registers

COC19

LDY	#$14		; Loop counter

LDX	#0		; Data Table Index

BACK1 	TXA

STA	AD6545		; 6545 Address = Register Number

LDA	HTOT,X		; Initial 6545 Register Data Values

STA	AD6545+1	; 6545 Register = Data

INX

DEY

BNE	BACK1

;	8.0 Exit

JMP	SEC		;set exit conditions, return

EJECT

SPACE	4,10

************************************************

**	SGB - Perform Screen GET-BYTE COMMAND **

************************************************

*

*	ENTRY	JSR	SGB

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

SGB	=	*	;entry

JSR	CCR	;check cursor range

JSR	GDC	;get data under cursor

JSR	CIA	;convert internal character to ATASCII

JSR	SZA	;set zero data and advance cursor

JMP	SST	;perform screen STATUS, return

EJECT

SPACE	4,10

**	GDC - Get Data under Cursor

*

*	ENTRY	JSR	GDC

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

*		2. Modified 5/7/84

*

GDC	=	*	;entry

JSR	CCA	;convert cursor row/column to address

LDA	(ADRESS),Y

GDC2	STA	CHAR

CMP	#0	;retore flags

RTS		;return

EJECT

SPACE	4,10

**************************************************

**	SPB - Perform Screen PUT-BYTE COMMAND	**

**************************************************

*

*	ENTRY	JSR	SPB

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

SPB	=	*	;entry

STA	ATACHR

JSR	ROD	;restore old data under cursor

CMP	#CLS

BNE	SPB1	;if not clear screen

JSR	CSC	;clear screen

JMP	SEC	;set exit conditions, return

SPB1	JSR	CCR	;check cursor range

;	JMP	CEL	;check EOL, return

EJECT

SPACE	4,10

**	CEL - Check End of Line

*

*	ENTRY	JSR	CEL

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

CEL	=	*	;entry

LDA	ATACHR

CMP	#EOL

BNE	CEL1	;if not EOL

JSR	RWS	;return with scrolling

JMP	SEC	;set exit conditions, return

CEL1	JSR	PLO	;plot point

JSR	SEA	;set EOL data and advance cursor

JMP	SEC	;set exit conditions, return

EJECT

SPACE	4,10

**	PLO - Plot Point

*

*	ENTRY	JSR	PLO

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

PLO	=	*		;entry

;	Wait for start/stop flag clear.

PLO0	LDA	SSFLAG		;start/stop flag

BNE	PLO0		;if start/stop flag non-zero, loop

;	Save cursor row/column.

LDX	#2		;offset to last byte

PLO1	LDA	ROWCRS,X	;byte of cursor row/column

STA	OLDROW,X	;save byte of cursor row/column

DEX

BPL	PLO1		;if not done

;	Convert ATASCII character to internal.

LDA	ATACHR		;character

TAY			;character

ROL	A

ROL	A

ROL	A

ROL	A

AND	#3

TAX			;index into TAIC

TYA			;character

AND	#$9F		;strip off column address

ORA	TAIC,X		;or in new column address

;	JMP	SPQ		;display, return

EJECT

SPACE	4,10

**	SPQ - Display

*

*	ENTRY	JSR	SPQ

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

*		2. Modified. 5/7/84

SPQ	=	*		;entry

;	Set CHAR.

STA	CHAR		;character

;	Convert cursor row/column to address.

JSR	CCA		;convert cursor row/column to address

;	Shift up to proper position.

LDA	CHAR		;character

STA	(ADRESS),Y	;update data

RTS			;return

EJECT

SPACE	4,10

**	SEC - Set Exit Conditions

*

*	ENTRY	JSR	SEC

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

SEC	=	*	;entry

JSR	GDC	;get data under cursor

STA	OLDCHR

LDX	DINDEX	;mode

BNE	SST	;if graphics, no cursor

LDX	CRSINH	;cursor inhibit flag

BNE	SST	;if cursor inhibited

EOR	#$80	;complement most significant bit

JSR	SPQ	;display

;	JMP	SST	;perform screen status, return

EJECT

SPACE	4,10

**************************************************

**	SST - Perform Screen   STATUS COMMAND	**

**		      Screen   CLOSE  COMMAND   **

**		      Editor   STATUS COMMAND	**

**		      Editor   CLOSE  COMMAND   **

**		      Keyboard OPEN   COMMAND	**

**		      Keyboard CLOSE  COMMAND	**

**		      Keyboard STATUS COMMAND	**

**************************************************

*

*	ENTRY	JSR	SST

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

SST	=	*

LDY	DSTAT	;status

SST1    LDA	#SUCCES	;indicate success

STA	DSTAT	;status

LDA	ATACHR	;data

;	JMP	ESP	;return

EJECT

SPACE	4,10

**************************************************

**	ESP - Perform Editor   SPECIAL COMMAND	**

**	              Keyboard SPECIAL COMMAND	**

**		      Screen   SPECIAL COMMAND  **

**************************************************

*

*	ESP does nothing.

*

*	ENTRY	JSR	ESP

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

ESP	=	*

RTS		;Return.

EJECT

SUBTTL	'EGB - PERFORM EDITOR GET-BYTE COMMAND'

SPACE	4,10

**************************************************

**	EGB - Perform Editor GET-BYTE COMMAND	**

**************************************************

*

*	ENTRY	JSR	EGB

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

EGB	=	*	;entry

;	Initialize.

JSR	SWA

JSR	CCR	;check cursor range for editor

LDA	BUFCNT	;buffer count

BNE	EGB4	;if something in the buffer

;	Get line.

LDA	ROWCRS		;cursor row

STA	BUFSTR		;buffer start pointer

LDA	COLCRS		;low cursor column

STA	BUFSTR+1	;high buffer start pointer

EGB1	JSR	KGB	;perform keyboard GET-BYTE

STY	DSTAT	;status

LDA	ATACHR	;ATASCII character

CMP	#EOL

BEQ	EGB3	;if EOL

JSR	PCH	;process character

JSR	SWA

LDA	LOGCOL	;logical column

CMP	#113	;column near column 120

BNE	EGB2	;if not near column 120, no beep

JSR	BEL	;beep

EGB2	JMP	EGB1	;process next character

;	Process EOL.

EGB3	JSR	ROD		;restore old data under cursor

JSR	CBC		;compute buffer count

LDA	BUFSTR		;buffer start pointer

STA	ROWCRS		;cursor row

LDA	BUFSTR+1	;high buffer start pointer

STA	COLCRS		;low cursor column

;	Check buffer count.

EGB4	LDA	BUFCNT	;buffer count

BEQ	EGB6	;if buffer count zero

;	Decrement and check buffer count.

EGB5	DEC	BUFCNT	;decrement buffer count

BEQ	EGB6	;if buffer count zero

;	Check status.

LDA	DSTAT	;status

BMI	EGB5	;if error, continue decrementing.

;	Perfrom GET-BYTE.

JSR	SGB	;perform screen GET-BYTE

STA	ATACHR	;ATASCII character

JMP	SWA	;swap, return

;	Exit.

EGB6	JSR	RWS	;return with scrolling

LDA	#EOL

STA	ATACHR	;ATASCII character

JSR	SEC	;set exit conditions

STY	DSTAT	;status

JMP	SWA	;swap, return

EJECT

SPACE	4,10

**	IRA - Invoke Routine Pointed to by ADRESS

*

*	ENTRY	JSR	IRA

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

IRA	=	*		;entry

JMP	(ADRESS)	;execute, return

EJECT

SPACE	4,10

**************************************************

**	EPB - Perform Editor PUT-BYTE COMMAND	**

**************************************************

*

*	ENTRY	JSR	EBP

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

EPB	=	*	;entry

STA	ATACHR	;ATASCII character

JSR	SWA	;swap

JSR	CCR	;check cursor range for editor

LDA	#0

STA	SUPERF	;clear super function flag

;	JMP	PCH	;process character, return

EJECT

SPACE	4,10

**	PCH - Process Character

*

*	PCH displays the character or processes control characters and

*	super functions (shifted function keys).

*

*	ENTRY	JSR	PCH

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

PCH	=	*	;entry

JSR	ROD	;restore old data under cursor

JSR	CCC	;check for control character

BEQ	PCH2	;if control character

;	Display character.

PCH1	ASL	ESCFLG	;escape flag

JSR	CEL	;check EOL

JMP	SWA	;swap, return

;	Process control character.

PCH2	LDA	DSPFLG	;display flag

ORA	ESCFLG	;escape flag

BNE	PCH1	;if dislay or escape, display character

;	Continue.

ASL	ESCFLG

INX

;	Check for super function.

LDA	SUPERF

BEQ	PCH3		;if not super function

;	Adjust for super function.

TXA

CLC

ADC	#TSFR-TCCR-3

TAX			;adjusted offset

;	Process control character or super function.

PCH3	LDA	TCCR,X		;low routine address

STA	ADRESS

LDA	TCCR+1,X	;high routine address

STA	ADRESS+1

JSR	IRA		;invoke routine pointed to by ADRESS

JSR	SEC		;set exit conditions

JMP	SWA		;swap, return

JMP	IGN

EJECT

SPACE	4,10

**	IGN - Ignore Character and Perform Keyboard GET-BYTE

*

*	ENTRY	JSR	IGN

*

*	EXIT

*		CH = $FF

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

IF	0

IGN	=	*

LDA	#$FF	;clear code indicator

STA	CH	;key code

;	JMP	KGB	;perform keyborad GET-BYTE, return

ENDIF

EJECT

SPACE	4,10

**************************************************

**	KGB - Perform Keyboard GET-BYTE COMMAND	**

**************************************************

*

*	ENTRY	JSR	KGB

*

*	NOTES

*		Problem: byte wasted by unnecessary TAX near KGB3.

*		KGB can reside in the os if these do also

*			IGN

*			SKC

*			CCC

*			SST

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

IF	0

KGB	=	*

;	Initialize.

KGB1	LDA	#0

STA	SUPERF	;clear super function flag

;	Check for special edit read mode.

LDA	ICAX1Z

LSR	A

BCS	KGB11	;if special edit read mode

;	Check for BREAK abort.

LDA	#BRKABT	;assume BREAK abort

LDX	BRKKEY	;BREAK key flag

BEQ	KGB10	;if BREAK abort

;	Check for character.

LDA	CH	;key code

CMP	#$FF	;clear code indicator

BEQ	KGB1	;if no character

;	Process character.

STA	HOLDCH	;save character

LDX	#$FF	;clear code indicator

STX	CH	;key code

;	Sound key click if desired.

LDX	NOCLIK	;click inhibit flag

BNE	KGB2	;if click inhibited

JSR	SKC	;sound key click

;	Set offset to key definition.

KGB2	TAY		;save character

;	Check for CTRL and SHIFT together.

CPY	#$C0

BCS	IGN	;if CTRL and SHIFT together, ignore

;	Convert to ATASCII character.

LDA	(KEYDEF),Y	;ATASCII character

;	Set ATASCII character.

KGB3	STA	ATACHR	;ATASCII character

TAX

BMI	KGB4	;if special key

JMP	KGB17	;process shift/control lock

;	Check for null character.

KGB4	CMP	#$80

BEQ	IGN	;if null, ignore

;	Check for inverse video key.

CMP	#$81

BNE	KGB5	;if not inverse video key

;	Process inverse video key.

LDA	INVFLG

EOR	#$80

STA	INVFLG

BCS	IGN	;ignore

;	Check for CAPS key.

KGB5	CMP	#$82

BNE	KGB6	;if not CAPS key

;	Process CAPS key.

LDA	SHFLOK	;shift/control lock flags

BEQ	KGB7	;if no lock, process CAPS lock

LDA	#$00	;no lock indicator

STA	SHFLOK	;shift/control lock flags

BEQ	IGN	;ignore

;	Check for SHIFT-CAPS key.

KGB6	CMP	#$83

BNE	KGB8	;if not SHIFT-CAPS

;	Process SHIFT-CAPS key.

KGB7	LDA	#$40	;CAPS lock indicator

STA	SHFLOK	;shift/control lock flags

BNE	IGN	;ignore

;	Check for CTRL-CAPS key.

KGB8	CMP	#$84

BNE	KGB9	;if not CTRL-CAPS

;	Process CTRL-CAPS key.

LDA	#$80	;control lock indicator

STA	SHFLOK	;shift/control lock flags

JMP	IGN	;ignore

;	Check for CTRL-3 key.

KGB9	CMP	#$85

BNE	KGB12	;if not CTRL-3 key.

;	Process CTRL-3 key.

LDA	#EOFERR

;	Set status and BREAK key flag.

KGB10	STA	DSTAT	;status

STA	BRKKEY	;BREAK key flag

;	Set EOL character.

KGB11	LDA	#EOL

JMP	KGB19	;set ATASCII character

;	Check for CTRL-F3 key.

KGB12	CMP	#$89

BNE	KGB14	;if not CTRL-F3 key

;	Process CTRL-F3 key.

LDA	NOCLIK	;toggle keyclick status

EOR	#$FF

STA	NOCLIK

BNE	KGB13	;if click inhibited

JSR	SKC	;sound key click

KGB13	JMP	IGN	;ignore

;	Check for function key.

KGB14	CMP	#$8E

BCS	KGB16	;if code >= $8E, not a function key

CMP	#$8A

BCC	KGB13	;if code < $8A, not a function key, ignore

;	Process function key.

SBC	#$8A		;convert $8A - $8D TO 0 - 3

ASL	HOLDCH		;saved character

BPL	KGB15		;if no SHIFT

ORA	#$04		;convert 0 - 3 to 4 - 7

KGB15	TAY			;offset to function key definition

LDA	(FKDEF),Y	;function key

JMP	KGB3		;set ATASCII character

;	Check for super function.

KGB16	CMP	#$92

BCS	KGB17	;if code >= $92, process shift/control lock

CMP	#$8E

BCC	KGB13	;if code < $8E, not super function, ignore

;	Process super function.

SBC	#$8E-$1C	;convert $8E - $91 TO $1C - $1F

INC	SUPERF		;set super function flag

BNE	KGB19		;set ATASCII character

;	Process shift/control lock.

KGB17	LDA	HOLDCH	;saved character

CMP	#$40

BCS	KGB18	;if not lower case

LDA	ATACHR	;ATASCII character

CMP	#'a'

BCC	KGB18	;if < "a", do not process

CMP	#'z'+1

BCS	KGB18	;if > "z", do not process

LDA	SHFLOK	;shift/control lock flags

BEQ	KGB18	;if no lock

ORA	HOLDCH	;modify character

JMP	KGB2	;reprocess character

;	Invert character, if necessary.

KGB18	JSR	CCC	;check for control character

BEQ	KGB20	;if control character, do not invert

LDA	ATACHR	;ATASCII character

EOR	INVFLG	;invert character

;	Set ATASCII character.

KGB19	STA	ATACHR	;ATASCII character

;	Exit.

KGB20	JMP	SST	;perform screen status, return

ENDIF

EJECT

SUBTTL	'Editor GET-BYTE Control Character and Super Function Routines.'

SPACE	4,10

**	ESC - Escape, Control Character Routine

*

*	ENTRY	JSR	ESC

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

ESC	=	*	;entry

LDA	#$80	;indicate escape detected

STA	ESCFLG	;escape flag

RTS		;return

EJECT

SPACE	4,10

**	CUP - Move Cursor Up, Control Character Routine

*

*	ENTRY	JSR	CUP

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

CUP	=	*	;entry

DEC	ROWCRS	;decrement cursor row

BPL	CUP2	;if row positive

LDX	BOTSCR	;screen bottom

DEX		;screen bottom - 1

CUP1	STX	ROWCRS	;update cursor row

CUP2	JMP	SBS	;set buffer start and logical column, return

EJECT

SPACE	4,10

**	CDN - Move Cursor Down, Control Character Routine

*

*	ENTRY	JSR	CDN

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

CDN	=	*	;entry

INC	ROWCRS	;increment cursor row

LDA	ROWCRS	;cursor row

CMP	BOTSCR	;screen bottom

BCC	CUP2	;if at bottom, set buffer start, return

LDX	#0

BEQ	CUP1	;update cursor row, return

EJECT

SPACE	4,10

**	CLF - Move Cursor Left, Control Character Routine

*

*	ENTRY	JSR	CLF

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

CLF	=	*	;entry

DEC	COLCRS	;decrement low cursor column

LDA	COLCRS	;low cursor column

BMI	CRM	;if negative, move cursor to margin, return

CMP	LMARGN	;left margin

BCS	SCC1	;if at left margin, set logical column, return

;	JMP	CRM	;move cursor to right margin, return

EJECT

SPACE	4,10

**	CRM - Move Cursor to Right Margin, Super Function Routine

*

*	ENTRY	JSR	CRM

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

CRM	=	*	;entry

LDA	RMARGN	;right margin

;	JMP	SCC	;set cursor column, return

EJECT

SPACE	4,10

**	SCC - Set Cursor Column

*

*	ENTRY	JSR	SCC

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

SCC	=	*	;entry

STA	COLCRS	;set low cursor column

SCC1	JMP	SLC	;set logical column, return

EJECT

SPACE	4,10

**	CRT - Move Cursor Right, Control Character Routine

*

*	ENTRY	JSR	CRT

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

CRT	=	*	;entry

INC	COLCRS	;increment low cursor column

LDA	COLCRS	;low cursor column

CMP	RMARGN	;right margin

BCC	SCC1	;if before right margin, process, return

BEQ	SCC1	;if at right margin

;	JMP	CLM	;move cursor to left margin, return

EJECT

SPACE	4,10

**	CLM - Move Cursor to Left Margin, Super Function Routine

*

*	ENTRY	JSR	CLM

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

CLM	=	*	;entry

LDA	LMARGN	;left margin

JMP	SCC	;set cursor column, return

EJECT

SPACE	4,10

**	CSC - Clear Screen, Control Character Routine

*

*	ENTRY	JSR	CSC

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

CSC	=	*		;entry

;	Set memory scan counter address.

JSR	SMS		;set memory scan counter address

;	Clear address.

LDY	ADRESS

LDA	#0

STA	ADRESS

CSC1	STA	(ADRESS),Y

INY

BNE	CSC1		;if not done with page

INC	ADRESS+1

LDX	ADRESS+1

CPX	RAMTOP		;(high) RAM size

BCC	CSC1		;if not done

;	Clean up logical line bit map.

;	LDY	#0		;offset to first byte of bit map

LDA	#$FF

CSC2	STA	LOGMAP,Y	;byte of logical line bit map

INY

CPY	#4		;4 bytes

BCC	CSC2		;if not done

;	Exit.

;	JMP	CHM		;move cursor home, return

EJECT

SPACE	4,10

**	CHM - Move Cursor Home, Super Function Routine

*

*	ENTRY	JSR	CHM

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

*		2. Modified. 5/7/84

CHM	=	*		;entry

JSR	SCL		;set cursor at left edge

STA	LOGCOL		;logical column

STA	BUFSTR+1	;high buffer start

LDA	#0

STA	ROWCRS		;cursor row

STA	BUFSTR		;low buffer start pointer

RTS			;return

EJECT

SPACE	4,10

**	BSP - Backspace, Control Character Routine

*

*	ENTRY	JSR	BSP

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

BSP	=	*	;entry

LDA	LOGCOL	;logical column

CMP	LMARGN	;left margin

BEQ	BSP3	;if at left margin

LDA	COLCRS	;low cursor column

CMP	LMARGN	;left margin

BNE	BSP1	;if not at left margin

JSR	DWQ	;see if line should be deleted

BSP1	JSR	CLF	;move cursor left

LDA	COLCRS	;low cursor column

CMP	RMARGN	;right margin

BNE	BSP2	;if not at right margin

LDA	ROWCRS	;cursor row

BEQ	BSP2	;if row zero

JSR	CUP	;move cursor up

BSP2	LDA	#' '

STA	ATACHR	;ATASCII character

JSR	PLO	;plot point

BSP3	JMP	SLC	;set logical column, return

EJECT

SPACE	4,10

**	TAB - Tab, Control Character Routine

*

*	ENTRY	JSR	TAB

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

TAB	=	*	;entry

TAB1	JSR	CRT	;move cursor right

LDA	COLCRS	;low cursor column

CMP	LMARGN	;left margin

BNE	TAB2	;if not at left margin

JSR	RET	;return

JSR	BLG	;get bit from logical line bit map

BCS	TAB3	;if end of logical line

;	Check for tab stop.

TAB2	LDA	LOGCOL	;logical column

JSR	BMG	;get bit from bit map

BCC	TAB1	;if not tab stop, keep looking

;	Set logical column.

TAB3	JMP	SLC	;set logical column, return

EJECT

SPACE	4,10

**	STB - Set Tab, Control Character Routine

*

*	ENTRY	JSR	STB

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

STB	=	*	;entry

LDA	LOGCOL	;logical column

JSR	BALT1   ;Logadr=logcol

JMP	BMS	;set bit in bit map, return

EJECT

SPACE	4,10

**	CTB - Clear Tab, Control Character Routine

*

*	ENTRY	JSR	CTB

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

CTB	=	*	;entry

LDA	LOGCOL	;logical column

JSR	BALT1

JMP	BMC	;clear bit in bit map, return

EJECT

SPACE	4,10

**	ICH - Insert Character, Control Character Routine

*

*	ENTRY	JSR	ICH

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

ICH	=	*	;entry

JSR	SRC	;save row and column

JSR	GDC	;get data under cursor

STA	INSDAT

LDA	#0

STA	SCRFLG

ICH1	JSR	SPQ	;store data

LDA	LOGCOL	;logical column

PHA		;save logical column

JSR	ACC	;advance cursor column

PLA		;saved logical column

CMP	LOGCOL	;logical column

BCS	ICH2	;if saved logical column >= logical column, exit

LDA	INSDAT

PHA

JSR	GDC	;get data under cursor

STA	INSDAT

PLA

JMP	ICH1	;continue

;	Exit.

ICH2	JSR	RRC	;restore row and column

ICH3	DEC	SCRFLG

BMI	ICH4	;if scroll occurred

DEC	ROWCRS	;decrement cursor row

BNE	ICH3	;continue

ICH4	JMP	SLC	;set logical column, return

EJECT

SPACE	4,10

**	DCH - Delete Character, Control Character Routine

*

*	ENTRY	JSR	DCH

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

DCH	=	*		;entry

;	Save row and column.

JSR	SRC		;save row and column

;	Get data to the right of cursor.

DCH1	JSR	CCA	;convert cursor row/column to address

LDA	ADRESS

STA	SAVADR	;save address

LDA	ADRESS+1

STA	SAVADR+1

LDA	LOGCOL	;logical column

PHA		;save logical column

JSR	SZA	;set zero data and advance cursor

PLA		;saved logical column

CMP	LOGCOL	;logical column

BCS	DCH2	;if saved logical column >= logical column, exit

LDA	ROWCRS		;cursor row

CMP	BOTSCR		;screen bottom

BCS	DCH2		;if row off screen, exit

JSR	GDC		;get data under cursor

LDY	#0

STA	(SAVADR),Y	;put data in previous position

BEQ	DCH1		;continue

DCH2	LDY	#0

TYA

STA	(SAVADR),Y	;clear last position

JSR	DQQ		;try to delete a line

JSR	RRC		;restore row and column

JMP	SLC		;set logical column, return

EJECT

SPACE	4,10

**	ILN - Insert Line, Control Character Routine

*

*	ENTRY	JSR	ILN

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

ILN	=	*	;entry

SEC

;	JMP	ILN1

EJECT

SPACE	4,10

**	ILN1 - Insert Line

*

*	ENTRY	JSR	ILN1

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

ILN1	=	*	;entry

JSR	ELL	;extend logical line

LDA	LMARGN	;left margin

STA	COLCRS	;low cursor column

JSR	CCA	;convert cursor row/column to address

JSR	MLN	;move line

JSR	CLN	;clear current line

JMP	SLC	;set logical column, return

EJECT

SPACE	4,10

**	DLN - Delete Line, Control Character Routine

*

*	ENTRY	JSR	DLN

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

DLN	=	*	;entry

JSR	SLC	;set logical column

LDY	HOLD1

STY	ROWCRS	;cursor row

;	JMP	DLN1

EJECT

SPACE	4,10

**	DLN1 - Delete Line

*

*	ENTRY	JSR	DLN1

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

DLN1	=	*		;entry

DLN0	LDY	ROWCRS		;cursor row

DLN2	TYA

SEC

JSR	BLG2		;get next bit

PHP

TYA

CLC

JSR	BALT		;Double byte ADC #8*[LOGMAP-TABMAP] ;add offset for logical line

PLP

JSR	BMP		;put bit in bit map

INY

CPY	#24

BNE	DLN2		;if not done

LDA	LOGMAP+2

ORA	#1		;set least significant bit

STA	LOGMAP+2	;update logical line bit map

LDA	#0		;delete line of data

STA	COLCRS		;low cursor column

JSR	CCA		;convert cursor row/column to address

JSR	SSD		;scroll screen for delete

;	Check for new logical line.

JSR	BLG		;get bit from logical line bit map

BCC	DLN0		;if not new logical line

;	Move cursor to left margin.

JMP	CLM		;move cursor to left margin, return

EJECT

SPACE	4,10

**	BEL - Sound Bell, Control Character Routine

*

*	ENTRY	JSR	BEL

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

BEL	=	*	;entry

LDY	#$20

BEL1	JSR	SKC	;sound key click

DEY

BPL	BEL1	;if not done

RTS		;return

EJECT

SPACE	4,10

**	CBT - Move Cursor to Bottom, Super Function Routine

*

*	ENTRY	JSR	CBT

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

CBT	=	*	;entry

JSR	CHM	;move cursor home

JMP	CUP	;move cursor up, return

EJECT

SPACE	4,10

**	CCA - Convert Cursor Row/Column to Address

*

*	ENTRY	JSR	CCA

*

*	MODS

*		L. Winner	06/01/82

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

*		2. Modified 5/7/84

CCA	=	*		;entry

;  	Addr = Savmsc + (2 * 2 * Rowcrs + Rowcrs) * 2 * 2 * 2 * 2 + Colcrs

LDX	#$00

STX	ADRESS+1	;clear high address

LDA	ROWCRS		;cursor row position

;	Calculate (2 * 2 * Rowcrs + Rowcrs).

ASL	A		;2 times row position

ROL	ADRESS+1

ASL	A		;4 time row position

ROL	ADRESS+1

ADC	ROWCRS		;add to get 5 times row position

STA	ADRESS

BCC	CCA1

INC	ADRESS+1

;	Calculate (Address * 2 * 2 * 2 * 2).

CCA1	LDX	#$04  		;left shift count for 80 columns

CCA2	ASL	ADRESS		;ADRESS = ADRESS * 2

ROL	ADRESS+1

DEX

BNE	CCA2

;	Calculate (Address + Colcrs).

CLC

LDA	COLCRS		;low cursor column

CCA4	ADC	ADRESS		;add address

BCC	CCA5		;if no carry

INC	ADRESS+1	;adjust high address

;	Calculate (Address + Savmsc).

CCA5	CLC

ADC	SAVMSC		;add saved memory scan counter

STA	ADRESS		;update address

STA	OLDADR		;save address

LDA	ADRESS+1

ADC	SAVMSC+1

STA	ADRESS+1

STA	OLDADR+1

;	Why?

LDY	#0

CCA6	RTS			;return

EJECT

SPACE	4,10

**	SZA - Set Zero Data and Advance Cursor Column

*

*	ENTRY	JSR	SZA

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

SZA	=	*	;entry

LDA	#0

BEQ	SDA	;set data and advance cursor

EJECT

SPACE	4,10

**	SEA - Set EOL Data and Advance Cursor Column

*

*	ENTRY	JSR	SEA

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

SEA	=	*	;entry

LDA	#EOL	;special case eliminator

;	JMP	SDA	;set data and advance cursor, return

EJECT

SPACE	4,10

**	SDA - Set Data and Advance Cusor Column

*

*	ENTRY	JSR	SDA

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

SDA	=	*	;entry

STA	INSDAT	;set data

;	JMP	ACC	;advance cursor column, return

EJECT

SPACE	4,10

**	ACC - Advance Cursor Column

*

*	ENTRY	JSR	ACC

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

*		2. Modified. 5/8/84

ACC	=	*		;entry

INC	LOGCOL		;increment logical column

INC	COLCRS		;increment low cursor column

ACC1	LDA	COLCRS		;low cursor column

LDX	DINDEX		;mode

CPX	#0		;if not mode 0, exit.

BNE	CCA6		;CCA6 is an RTS.

CMP	#80     	;Mode column count is always 80

BEQ	ACC2		;if equal, process EOL

CMP	RMARGN		;right margin

BEQ	CCA6		;if at right margin, exit

BCC	CCA6		;if before right margin, exit

ACC2	JMP	RET		;exit

EJECT

SPACE	4,10

**	RWS - Return with Scrolling

*

*	ENTRY	JSR	RWS

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

RWS	=	*	;entry

LDA	#EOL	;select scrolling

STA	INSDAT

;	JMP	RET	;return, return

EJECT

SPACE	4,10

**	RET - Return

*

*	ENTRY	JSR	RET

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

*		2. Modified. 5/7/84

RET	=	*		;entry

JSR	SCL		;set cursor at left edge

INC	ROWCRS		;increment cursor row

LDX	DINDEX

LDY	#24		;assume 24 lines

TYA	     		;mode row count

CMP	ROWCRS		;cursor row

BNE	RET5

STY	HOLD3

TXA			;mode

BNE	RET5		;if mode not 0, do not scroll

LDA	INSDAT

BEQ	RET5	;if zero, do not scroll

;	If EOL, roll in a 0.

CMP	#EOL	;to extend bottom logical line

BEQ	RET3	;if EOL

CLC

RET3	JSR	SCR

INC	SCRFLG

DEC	BUFSTR

BPL	RET4

INC	BUFSTR

RET4	DEC	HOLD3

LDA	LOGMAP

SEC		;indicate for partial line

BPL	RET3	;if partial logical line

LDA	HOLD3

STA	ROWCRS	;cursor row

RET5	JMP	SLC	;set logical column, return

EJECT

SPACE	4,10

**	CCR - Check Cursor Range

*

*	ENTRY	JSR	CCR

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

CCR	=	*		;entry

LDA	#79

CMP	RMARGN		;right margin

BCS	CCR1		;if 79 >= right margin

STA	RMARGN		;set right margin

CCR1    LDX	DINDEX

LDA	#24   		;mode row count

CMP	ROWCRS		;cursor row

BCC	CCR5		;if count > row position, error

BEQ	CCR5		;if count = row position, error

CCR3	LDA	#79     	;mode column count

CMP	COLCRS		;low cursor column

BCC	CCR5		;if count > column position, error

BEQ	CCR5		;if count = column position, error

CCR4	LDA	#SUCCES		;success indicator

STA	DSTAT		;indicate success

LDA	#BRKABT		;assume BREAK abort

LDX	BRKKEY		;BREAK key status

STA	BRKKEY		;clear BREAK key status

BEQ	CCR6		;if BREAK

RTS			;return

;	Process range error.

CCR5	JSR	CHM		;move cursor home

LDA	#CRSROR		;indicate cursor overrange

;	Exit.

CCR6	STA	DSTAT		;status

PLA			;clean stack for return to CIO

PLA

CCR7	JMP	SST		;return (to CIO)

EJECT

SPACE	4,10

**	ROD - Restore Old Data under Cursor

*

*	ENTRY	JSR	ROD

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

ROD	=	*		;entry

LDY	#0

LDA	OLDADR+1

BEQ	ROD1		;if page zero

LDA	OLDCHR		;old data

STA	(OLDADR),Y

ROD1	RTS			;return

EJECT

SPACE	4,10

**	BIT MAP ROUTINES         **

BMI	=	*		;entry

LDA	LOGADR+1

;	Set BITMSK

AND	#7		;logical column modulo 8

TAX			;offset to bit mask

LDA	TBTM,X		;bit mask (from TBTM).

STA	BITMSK		;set bit mask

;	byte index

LDX	#3

BMI1	LSR	LOGADR		;divide by 2

ROR	LOGADR+1

DEX

BNE	BMI1

LDA	LOGADR+1

TAX			;offset

RTS			;return

**	BLR - Rotate Logical Line Bit Map Left

BLR	ROL	LOGMAP+2

ROL	LOGMAP+1

ROL	LOGMAP

RTS			;return

**	Put Bit

BMP	BCC	BMC		;if C clear, clear bit in bit map, return

BMS	JSR	BMI		;initialize for bit mask operation

LDA	TABMAP,X

ORA	BITMSK		;set bit

STA	TABMAP,X	;update bit map

RTS			;return

**	Clear Bit

BMC	JSR	BMI		;initialize for bit mask operation

LDA	BITMSK

EOR	#$FF

AND	TABMAP,X	;clear bit

STA	TABMAP,X	;update bit map

RTS			;return

**	Get Bit

BLG	LDA	ROWCRS			;cursor row

BLG1	CLC

BLG2	JSR	BALT			;double byte ADC #8*[LOGMAP-TABMAP]

BMG	JSR	BMI			;initialize for bit mask operation

CLC

LDA	TABMAP,X

AND	BITMSK

BEQ	BMG1

SEC

BMG1	RTS				;return

**	BALT

*

*	Logadr = 6502A + 8*[logmap - tabmap]

*

BALT	PHP				;Save the C flag

JSR	BALT4

PLP

JSR	BALT2

RTS

BALT1   LDX	#$00

STX	LOGADR+1

STX	LOGADR

CLC

BALT2   ADC	LOGADR+1                ;plus 6502 A register

STA	LOGADR+1

BCC	BALT3

INC	LOGADR

BALT3	RTS

BALT4   LDX	#[LOGMAP - TABMAP]	;Low value

STX	LOGADR+1

LDX	#0

STX	LOGADR			;High value

LDX	#3

BALT41	ASL	LOGADR+1		;times 2

ROL	LOGADR

DEX

BNE	BALT41

BALT5	RTS

EJECT

SPACE	4,10

**	CIA - Convert Internal Character to ATASCII

*

*	ENTRY	JSR	CIA

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

*		2. 6545

*		   R. L. Blotcky 05/08/84

CIA	=	*	;entry

;	Initialize.

LDA	CHAR

;	Convert internal character to ATASCII.

CIA1	ROL	A

ROL	A

ROL	A

ROL	A

AND	#3

TAX

LDA	CHAR	;character

AND	#$9F	;strip off cloumn address

ORA	TIAC,X	;or in new column address

;	Exit.

CIA2	STA	ATACHR	;ATASCII character

CIA3	RTS		;return

EJECT

SPACE	4,10

**	MLN - Move Line

*

*	ENTRY	JSR	MLN

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

MLN	=	*		;entry

;	Initialize for 80 character lines.

LDX	RAMTOP		;(high) RAM size

DEX			;decrement (high) RAM size

STX	FRMADR+1	;high source address

STX	TOADR+1		;high destination address

LDA	#low [$0000-160]	;low RAM size - 160

STA	FRMADR		;low source address

LDA	#low [$0000-80]	;low RAM size - 80

STA	TOADR		;low destination address

LDX	ROWCRS		;cursor row

;	Check for completion.

MLN1	INX

CPX	BOTSCR		;screen bottom

BEQ	BALT5		;if done, return

;	Move line of 80 characters.

LDY	#79		;offset to last byte

MLN2	LDA	(FRMADR),Y	;byte of source

STA	(TOADR),Y	;byte of destination

DEY

BPL	MLN2		;if not done

;	Adjust source and destination addresses.

SEC

LDA	FRMADR		;source address

STA	TOADR		;update destination address

SBC	#low 80		;subtract 80

STA	FRMADR		;update source address

LDA	FRMADR+1

STA	TOADR+1

SBC	#high 80

STA	FRMADR+1

JMP	MLN1		;continue

EJECT

SPACE	4,10

**	ELL - Extend Logical Line

*

*	ENTRY	JSR	ELL

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

ELL	=	*	;entry

PHP		;save bit

LDY	#22

ELL1	TYA

JSR	BLG1

PHP

TYA

CLC

JSR	BALT		;Double byte ADC #8*[LOGMAP-TABMAP]+1	;add offset for logical line

PLP

JSR	BMP	;put bit in bit map

DEY

BMI	ELL2

CPY	ROWCRS	;cursor row

BCS	ELL1

ELL2	LDA	ROWCRS	;cursor row

CLC

JSR	BALT		;Double byte ADC #8*[LOGMAP-TABMAP]	;add offset for logical line

PLP

JMP	BMP	;put bit in bit map, return

EJECT

SPACE	4,10

**	CLN - Clear Line

*

*	ENTRY	JSR	CLN

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

CLN	=	*	;entry

LDA	LMARGN	;left margin

STA	COLCRS	;low cursor column

JSR	CCA	;convert cursor row/column to address

SEC

LDA	RMARGN	;right margin

SBC	LMARGN	;subtract left margin

TAY		;screen width

LDA	#0

CLN1	STA	(ADRESS),Y

DEY

BPL	CLN1	;if not done

RTS		;return

EJECT

SPACE	4,10

**	SCR - Scroll

*

*	ENTRY	JSR	SCR

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

SCR	=	*	;entry

;	Initialize.

JSR	BLR	;rotate logical line bit map left

;	Exit.

SCR5	JSR	SMS	;set memory scan counter address

;	JMP	SSD	;scroll screen for delete, return

EJECT

SPACE	4,10

**	SSD - Scroll Screen for Delete

*

*	ENTRY	JSR	SSD

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

SSD	=	*		;entry

;	Initialize.

LDA	ADRESS		;low address

LDX	ADRESS+1        ;high address

;	Calculate number of bytes to move.

SSD1	INX   			;high address

CPX	RAMTOP

BEQ	SSD2		;if at RAMTOP

SEC

SBC	#$10

JMP	SSD1		;continue

SSD2	ADC	#79		;(CLC and ADC #80)

BNE	SSD3		;if byte count non-zero

LDX	ADRESS+1

INX

CPX	RAMTOP

BEQ	SSD6		;if at RAMTOP

CLC

ADC	#$10

;	Adjust address.

SSD3	TAY			;number of bytes

STA	COUNTR

SEC

LDA	ADRESS

SBC	COUNTR		;subtract

STA	ADRESS		;update low address

BCS	SSD4		;if no borrow

DEC	ADRESS+1	;adjust high address

;	Move data down.

SSD4	LDA	ADRESS

CLC

ADC	#80

STA	COUNTR		;address + 80

LDA	ADRESS+1

ADC	#0

STA	COUNTR+1

SSD5	LDA	(COUNTR),Y	;byte to move

STA	(ADRESS),Y	;move byte

INY

BNE	SSD5		;if not done (256-16 times)

LDY	#256-240

LDA	ADRESS

CMP	#-80            ;80 columns

BEQ	SSD6		;if all done

CLC

ADC	#240

STA	ADRESS		;update low address

BCC	SSD4		;if no carry

INC	ADRESS+1	;adjust high address

BNE	SSD4		;continue

;	Clear last line.

SSD6	LDX	RAMTOP

DEX

STX	COUNTR+1

LDX	#-80

STX	COUNTR

LDA	#0

LDY	#79

SSD7	STA	(COUNTR),Y	;clear byte of last line

DEY

BPL	SSD7		;if not done

;	JMP	SLC		;set logical column, return

EJECT

SPACE	4,10

**	SLC - Set Logical Column

*

*	ENTRY	JSR	SLC

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

SLC	=	*	;entry

;	Initialize.

LDA	#0

STA	LOGCOL	;initialize logical column

LDA	ROWCRS	;cursor row

STA	HOLD1	;working row

;	Search for beginning of line.

SLC1	LDA	HOLD1	;add in row component

JSR	BLG1

BCS	SLC2	;if beginning of line found

LDA	LOGCOL	;logical column

CLC

ADC	#80	;add number of characters per line

STA	LOGCOL	;update logical column

DEC	HOLD1	;decrement working row

JMP	SLC1	;continue

;	Add in cursor column.

SLC2	CLC

LDA	LOGCOL	;logical column

ADC	COLCRS	;add low cursor cloumn

STA	LOGCOL	;update logical column

RTS		;return

EJECT

SPACE	4,10

**	CBC - Compute Buffer Count

*

*	CBC computes the buffer count as the number of bytes from the

*	buffer start to the end of the logical line (with trailing

*	spaces removed).

*

*	ENTRY	JSR	CBC

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

*		2. Modified. 5/7/84

CBC	=	*	;entry

;	Initialize.

JSR	SRC		;save row and column

LDA	LOGCOL		;logical column

PHA			;save logical column

LDA	BUFSTR		;start of buffer

STA	ROWCRS		;cursor row

LDA	BUFSTR+1

STA	COLCRS		;low cursor column

LDA	#1

STA	BUFCNT		;initialize buffer count

;	Determine last line on screen.

CBC1	LDX	#23		;normal last line on screen

;	Check for cursor on last line of screen.

CBC2	CPX	ROWCRS	;cursor row

BNE	CBC3	;if cursor on last line

LDA	COLCRS	;low cursor column

CMP	RMARGN	;right margin

BNE	CBC3	;if not at right margin

INC	BUFCNT	;fake SEA to avoid scrolling

JMP	CBC4

CBC3	JSR	SZA	;set zero data and advance cursor

INC	BUFCNT

LDA	LOGCOL	;logical column

CMP	LMARGN	;left margin

BNE	CBC1	;if not yet at left margin

DEC	ROWCRS	;decrement cursor row

JSR	CLF	;move cursor left

CBC4	JSR	GDC	;get data under cursor

BNE	CBC6	;if non-zero, quit

DEC	BUFCNT	;DECREMENT COUNTER

LDA	LOGCOL	;logical column

CMP	LMARGN	;left margin

BEQ	CBC6	;if beginning of logical line, exit

JSR	CLF	;move cursor left

LDA	COLCRS	;low cursor column

CMP	RMARGN	;right margin

BNE	CBC5	;if cursor column not right margin

DEC	ROWCRS	;decrement cursor row

CBC5	LDA	BUFCNT

BNE	CBC4	;if BUFCNT non-zero, continue

CBC6	PLA		;saved logical column

STA	LOGCOL	;restore logical column

JMP	RRC	;restore row and column, return

EJECT

SPACE	4,10

**	SBS - Set Buffer Start and Logical Column

*

*	ENTRY	JSR	SBS

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

SBS	=	*		;entry

JSR	SLC		;set logical column

LDA	HOLD1

STA	BUFSTR

LDA	LMARGN		;left margin

STA	BUFSTR+1

SBS1	RTS			;return

EJECT

SPACE	4,10

**	DQQ - Delete Line

*

*	ENTRY	JSR	DQQ

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

DQQ	=	*	;entry

LDA	LOGCOL	;logical column

CMP	LMARGN	;left margin

BNE	DQQ1	;if not at left margin

DEC	ROWCRS	;decrement cursor row

DQQ1	JSR	SLC	;set logical column

;	JMP	DWQ

EJECT

SPACE	4,10

**	DWQ - Delete Line

*

*	ENTRY	JSR	DWQ

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

DWQ	=	*	;entry

;	Check for left margin.

LDA	LOGCOL	;logical column

CMP	LMARGN	;left margin

BEQ	SBS1	;if at left margin, return

JSR	CCA	;convert cursor row/column to address

LDA	RMARGN	;right margin

SEC

SBC	LMARGN	;subtract left margin

TAY		;offset to last byte

DWQ1	LDA	(ADRESS),Y

BNE	SBS1

DEY

BPL	DWQ1	;if not done

JMP	DLN1	;delete line, return

EJECT

SPACE	4,10

**	CCC - Check for Control Character

*

*	ENTRY	JSR	CCC

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

IF	0

CCC	=	*      		;os entry point

LDX	#TCCRL-3	;offset to last entry

CCC1	LDA	TCCR,X		;control character

CMP	ATACHR		;ATASCII character

BEQ	CCC2		;if character found, exit

DEX

DEX

DEX

BPL	CCC1		;if not done, continue search

CCC2	RTS			;return

ENDIF

EJECT

SPACE	4,10

**	SRC - Save Row and Column

*

*	ENTRY	JSR	SRC

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

SRC	=	*		;entry

LDX	#2		;offset to last byte

SRC1	LDA	ROWCRS,X	;byte of cursor row/column

STA	TMPROW,X	;save byte of cursor row/column

DEX

BPL	SRC1		;if not done

RTS			;return

EJECT

SPACE	4,10

**	RRC - Restore Row and Column

*

*	ENTRY	JSR	RRC

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

RRC	=	*		;entry

LDX	#2		;offset to last byte

RRC1	LDA	TMPROW,X	;byte of saved cursor row/column

STA	ROWCRS,X	;byte of cursor row/column

DEX

BPL	RRC1		;if not done

RTS			;return

EJECT

SPACE	4,10

**	SWA - Swap Cursor Position with Regular Cursor Position

*

*	ENTRY	JSR	SWA

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

*		2. Modified. 5/7/84

SWA	=	*		;entry

JMP	SST	;perform screen STATUS, return

EJECT

SPACE	4,10

**	SKC - Sound Key Click

*

*	ENTRY	JSR	SKC

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

IF	0

SKC	=	*

;	Initialize.

LDX	#2*63	;2 times trip count

PHA		;save A

;	Turn loudspeaker on.

SKC1	STX	CONSOL

;	Wait for VBLANK (loudspeaker off).

LDA	VCOUNT	;vertical line counter

SKC2	CMP	VCOUNT	;current vertical line counter

BEQ	SKC2	;if vertical line not changed

;	Decrement and check trip count.

DEX

DEX

BPL	SKC1	;if not done

;	Exit.

PLA		;restore A

RTS		;return

ENDIF

EJECT

SPACE	4,10

**	SCL - Set Cursor at Left Edge

*

*	ENTRY	JSR	SCL

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

*		2. Modified. 5/7/84

SCL	=	*	;entry

;	Set Colcrs to Lmargn.

LDA	LMARGN	;use left margin

STA	COLCRS	;set low cursor column

RTS		;return

EJECT

SPACE	4,10

**	SMS - Set Memory Scan Counter Address

*

*	ENTRY	JSR	SMS

*

*	MODS

*		Original Author Unknown

*		1. Bring closer to Coding Standard (object unchanged).

*		   R. K. Nordin	11/01/83

SMS	=	*		;entry

LDA	SAVMSC		;saved low memory scan counter

STA	ADRESS		;set low address

LDA	SAVMSC+1	;saved high memory scan counter

STA	ADRESS+1	;set high address

RTS			;return

EJECT

SUBTTL	'TABLES'

SPACE	4,10

**	TCCR - Table of Control Character Routines

*

*	Each entry is 3 bytes.  The first byte is the control

*	character; the second and third bytes are the address of

*	the routine which processes the control character.

TCCR	DB	$1B

DW	ESC	;escape

DB	$1C

DW	CUP	;move cursor up

DB	$1D

DW	CDN	;move cursor down

DB	$1E

DW	CLF	;move cursor left

DB	$1F

DW	CRT	;move  cursor right

DB	$7D

DW	CSC	;clear screen

DB	$7E

DW	BSP	;backspace

DB	$7F

DW	TAB	;tab

DB	$9B

DW	RWS	;return with scrolling

DB	$9C

DW	DLN	;delete line

DB	$9D

DW	ILN	;insert line

DB	$9E

DW	CTB	;clear tab

DB	$9F

DW	STB	;set tab

DB	$FD

DW	BEL	;sound bell

DB	$FE

DW	DCH	;delete character

DB	$FF

DW	ICH	;insert character

TCCRL	=	*-TCCR	;length

SPACE	4,10

**	TSFR - Table of Super Function (Shifted Function Key) Routines

*

*	Each entry is 3 bytes.  The first byte is the super function

*	character; the second and third bytes are the address of the

*	routine which processes the super function.

TSFR	DB	$1C

DW	CHM	;move cursor home

DB	$1D

DW	CBT	;move cursor to bottom

DB	$1E

DW	CLM	;move cursor to left margin

DB	$1F

DW	CRM	;move cursor to right margin